3GPP TSG-SA2 Meeting #154	S2-2210813

# Toulouse, France, November 14th -18th , 2022												(new CR)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0790 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support binding an NF request targeting a group of UE(s) to the relevant PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the FS_AIMLsys study conclusion in SA2#153e meeting, ### 8.6.2 QoS request for a group of UEs For AIML each of the members of the group has a PDU Session established, and the members of the group have been selected using procedures defined in conclusions to KI#7, such as location or QoS monitoring. In order to request QoS for the AIML communication with each of the members of the group, extensions to the procedure for the AF request with QoS, and Nnef_AFsessionwithQoS to provide a list of UE IP address are required. The QoS parameters that are provided apply for each of the UE to AIML application communication (i.e. the QoS parameters are not related to the aggregated QoS for all FL members), as such those are provided to the PCF in the Npcf_PolicyAuthorization. For the scenario where there are more than one PCFs serving the list of UEs whose group QoS is requested, extensions to BSF management discovery to provide a list of PCF addresses based on the UE addresses are supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause added to Binding an NF request targeting a group of UE(s) to the relevant PCF that match each UE in the group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for BSF discovery to provide a list of PCF addresses based on the UE addresses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.1.1.2x	Binding an NF request targeting a group of UE(s) to the relevant PCF that match each UE in the group

When multiple and separately addressable PCFs have been deployed, a network functionality is required in order to ensure that a consumer NF (e.g. AF, 5G DDNMF) reaches the PCF serving the UE. This network functionality is provided by the BSF (described in clause 6.1.1.2.2). and has the characteristics as described in clause 6.1.1.2.2a. If the consumer NF (e.g. the AF or the NEF) provides list of UE address(es), the BSF shall provide the address of a PCF for a PDU Session mapping to each UE address provided by the consumer NF.

* * * End of Changes * * *