3GPP TSG-WG SA2 Meeting #154 	S2-2210814

Toulouse, France, Nov 14 – 18, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3649 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AFs to request time synchronization service in a specific coverage area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Reflect the conclusion about how to enable AFs to request time synchronization service in a specific coverage area in TR23.700-25 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Clause 4.15.9.1 - Introduce general information about AFs to request time synchronization service 2.Clause 4.15.9.X - Introduce how to enable AFs to request time synchronization service in a specific coverage area |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 TS 23.501 does not involve the AFs to request time synchronization service in a specific coverage area |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.1, 4.15.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.15.9.1	General

Time synchronization exposure allows an AF to configure time synchronization in 5GS. For (g)PTP operation, the Time synchronization service allows an AF to subscribe to the UE and 5GC capabilities and availability for time synchronization service (as described in clause 4.15.9.2) and to configure the (g)PTP instance in 5GS as described in clause 4.15.9.3. For 5G access stratum based time distribution, the AF can influence the 5G access stratum time distribution as described in clause 4.15.9.4. For AFs to request time synchronization service in a specific coverage area as described in clause 4.15.9.X. The time synchronization exposure is provided by NEF that uses the service provided by TSCTSF. The AF that is part of operator's trust domain may invoke the services directly with TSCTSF and TSCTSF responds/notifies directly to the AF, accordingly.

NOTE:	The AF can use either the procedure for configuring the (g)PTP instance in 5GS as described in clause 4.15.9.3 or the procedure for controlling the 5G access stratum time distribution as described in clause 4.15.9.4 for a particular UE. The procedures are not intended to be used in conjunction with each other by the AF. However, the (g)PTP instance activation, modification, and deactivation can influence the 5G AS time distribution for the UEs that are part of the impacted PTP instance.

* * * Second Change (all new text) * * *

4.15.9.X	Procedures for AFs to request time synchronization service in a specific coverage area

The procedure depicted in Figure 4.15.9.X-1 for the AF requested time synchronization service in a specific coverage area is illustrated as follows.

Figure 4.15.9.X-1: AF requesting time synchronization service in a specific area

1.	AM policy association establishment is finalized during UE registration procedure.

2.	The procedure is triggered by the AF request to influence the 5G time distribution. The coverage area information is added in step 2 when the AF requests the service in a specific area.

3.	The NEF authorizes the request and invokes the operation with the corresponding TSCTSF. TSCTSF calculates the Uu time synchronization error budget if the AF provides the error budget requirements. The coverage area information is considered together with the 5G access stratum time distribution indication by the TSCSTF. If 5G access stratum time distribution is enabled in the coverage area, it can meet the request. If another gNB not in the coverage area or time-synchronized UPF/NW-TT can meet the request requirements, it provides reference time to the targeted UE in the area to enforce AF request if possible.

4.	If the 5G access stratum time distribution parameters in UDR are associated with a DNN/S-NSSAI for the PCF for the UE may discover the PCF for the PDU Session using SUPI and (DNN, S-NSSAI) as parameters.

5.	TSCTSF selects for the PCF that handles the AM Policy Association of the targeted UE with SUPI (step 4) as an input parameter and informs the PCF to consider UE location information when determining AM policy.

6.	If the TSCTSF sends multiple time synchronization error budgets for a given UE, the PCF would pick the most stringent budget. The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure. In the step, the AMF is responsible for reporting UE location information to PCF. When PCF receives UE location information, it determines to provide which 5GS access stratum time information to UE based on time synchronization date sent by TSCTSF. As part of this, the 5G access stratum time distribution indication and the Uu time synchronization error budget are provided to NG-RAN. Based on this, NG-RAN provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

7.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthenorization operation.

8.	The TSCTSF responds to the NEF request in step 3.

9.	The NEF informs the AF about the result of the operation in step 2.

* * *End of Changes * * *