3GPP TSG-SA WG2 Meeting #154	S2-2210868

# Nov 14 – 18, 2022, Toulouse

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0262 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Positioning Reference Units |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  |  | Date: |  | 2022-10-14 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | - TR 23.700-71 concluded, among others, below related to the PRU KI#7: For PRU Registration, the conclusions include: - UE sends UL NAS Transport including PRU Registration Request to AMF. - The PRU Registration is an LCS supplementary service message and has no new AMF impact. - AMF sends the PRU Registration Request to LMF and may include a UE verification indication indicating whether this UE can serve as a PRU. - PRU Registration Request includes PRU information. - PRU information may include PRU's positioning capability, location information (if known), type and state of the PRU (e.g. mobile/static type, on/off state) - LMF may indicate to NRF the PRU information or may store PRU information locally based on operator policy. - LMF stores PRU information and may indicate to NRF the PRU existence in its NF profile. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR updates TS 23.273 to include PRU related agreements in the TR 23.700-71, in particular on PRU registration agreements |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Positioning Reference Units in TS 23.273 |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.X.  Support of Positioning Reference Units

A Position Reference Unit (PRU) needs to register with the network to get authorized to receive services, and to enable reachability, etc. The PRU may initiate the Registration procedure as Initial Registration, Periodic Registration update and Mobility Registration Update upon changing to a new Tracking Area (TA) outside the UE's Registration Area.

Below figure 6.X-1 provides detail PRU registration procedure.

Figure 6.X-1: PRU registration procedure

- PRU sends Registration request to (R)AN, indicating in the NAS Registration Request (to the AMF) its PRU functionality, and PRU information (positioning capabilities, location information (if known), static/mobile).

- (R)AN selects AMF as defined in TS 23.502 clause 4.2.2.2.2

- This is same as Steps 3-19 as in TS 23.502, clause 4.2.2.2.2

- On successful registration, AMF sends Registration Accept message to the PRU

- This is same as steps 21-25 as in TS 23.502, clause 4.2.2.2.2

- AMF selects LMF e.g. based on service area, and other criteria as in TS 23.273

- AMF provides PRU information to the LMF. AMF may send PRU information in the Nlmf_Location_DetermineLocation request message to the LMF e.g. in case AMF determines need to determine accurate PRU location (if not known already).

- AMF/LMF may initiate NI-LR procedure to determine accurate PRU location e.g. if not already known

- LMF updates its LMFProfile in the NRF, to include PRU information.

Steps 6 and 7 may be done for multiple LMF(s)

* * * * End of Changes * * * *