3GPP TSG-WG SA2 Meeting #154 	S2-2210876

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Apple

Title:	KI#1: Conclusions update

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: Resolved ENs in conclusions for key issue #1.

# 1. Discussion

It is proposed to resolve the ENs as following:

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-41:

# * * * * First change * * * *

## 8.1	Conclusions for KI#1

The following principles are concluded for the normative work:

NOTE 1:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on AM-PCF notification.

NOTE 2: The NF service between AMF and AM-PCF will be defined during the normative phase.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the AM-PCF and/or with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

2)	In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

NOTE 3: 	A new information element will be specified during the normative phase to include the association between the alternative S-NSSAI and the old S-NSSAI.

If there are existing PDU Sessions on the old S-NSSAI, the AMF includes in the Allowed NSSAI the alternative S-NSSAI and temporarily the old S-NSSAI to ensure that first the PDU Session transfer to the alternative S-NSSAI is performed before releasing the PDU Session on the old S-NSSAI.

The URSP rules are not updated with the alternative S-NSSAI, consequently, the UE may establish a new PDU session to the old S-NSSAI that will be transferred to the alternative S-NSSAI as per 2b).

-

b)	For a new PDU Session establishment request from the UE on the old S-NSSAI, the AMF proceeds with the PDU Session establishment to the SMF and indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI.

NOTE 4:	If the AMF already sent UE MM configuration update to the UE including the alternative S-NSSAI as per 2a), the UE should not send new PDU Session establishment request using the old S-NSSAI.

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and optionally indicates the alternative S-NSSAI. In home routed roaming case, the AMF notifies the H-SMF via the V-SMF to relocate the PDU Session to the alternative HPLMN S-NSSAI.

-	The SMF sends either a PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) to the UE and indicates that the PDU Session can be re-established on another S-NSSAI. If AMF indicated the alternative S-NSSAI to the SMF, the SMF provides the alternative S-NSSAI to the UE.

-	The UE triggers a new PDU Session Establishment procedure to establish a PDU Session using the alternative S-NSSAI.

--

# * * * * End of changes * * * *