3GPP TSG-SA2 Meeting #154	S2-2210884

# Toulouse, France, Nov 14 - 18, 2022						 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0153 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Negotiated Layer-3 based UE-to-UE Relay Re-selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In study, it was concluded to include L3 U2U Relay reselection using sol#7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces the solution to negotiated L3-based UE-to-UE Relay reselection procedure based on KI #1 conclusion in TR 23.700-33. - For UE-to-UE Relay reselection, the negotiated UE-to-UE Relay reselection between Source UE and Target UE in Sol#7 and the UE-to-UE Relay selection procedure in Sol#10 can be used under different conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relay reselection based on TR conclusion will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X or 6.X.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes – ALL TEXT IS NEW * * *

## 6.x	ProSe UE-to-UE Relay Communication

### 6.x.y  Layer-3 based UE-to-UE Relay Re-selection

Figure 6.x.y-1 illustrates the procedure for UE-to-UE Relay re-selection for Layer-3 based solution.

Figure 6.x.y-1: Layer-3 based UE-to-UE Relay Re-selection

1.	Source UE has a PC5 unicast link established with Relay 1.

1a.	Target UE has a PC5 unicast link established with Relay 1.

2.	Source UE has a PC5 unicast link established with Relay 2.

2a.	Target UE has a PC5 unicast link established with Relay 2.

3.	Source UE and Target UE exchange IP packets via Relay 1.

4.	Source UE detects or receives a trigger to change the Relay.

5.	Source UE sends a PC5 Link Modification Request message for Target UE (via the PC5 unicast link with Relay 1) which includes a Relay reselection indication, a list of candidate Relay IDs (RIDs), Source UE and Target UE IP addresses used to communicate via Relay 1. The IP addresses are specified since the Relay need them to fetch the corresponding PC5 unicast link from its mapping table toward the Target UE.

The list of candidate RIDs is selected based on the PC5 unicast links already established between Source UE and such Relays (RIDs) and/or based on the discovered relays. The IP address of multiple Target UEs may be specified in the case where Source UE is communicating with multiple Target UEs via Relay 1.

6.	Relay 1 determines that the message is destined to the Target UE based on the "Relay reselection indication" and Target UE's IP address. Relay 1 sends a PC5 Link Modification Request message to Target UE's IP address (as specified in the message); if multiple Target UEs are impacted, Relay 1 sends a PC5 Link Modification Request to each of these Target UEs.

7.	Based on the Relay reselection indication, Target UE knows that the communication with Source UE needs to be handled via another Relay. Target UE selects a RID from the list of candidate RIDs. Target UE may select a RID with which a PC5 unicast link is already established, or Target UE may establish a new PC5 unicast link with the selected RID.

8.	Target UE sends a Link Modification Accept message to Relay 1, which includes a Relay reselection indication, Source UE and Target UE IP addresses used via Relay 1, Target UE's IP address used via Relay 2 and the selected RID (e.g. RID2).

9.	Relay 1, based on the relay reselection indication, sends a PC5 Link Modification Accept message to Source UE using the specified Source UE's IP address.

10.	Source UE extracts the selected Relay ID (e.g. RID2) from the Link Modification Accept message and sends a Link Modification Ack message, which includes its IP address associated to Relay 2, to Relay 1.

11.	Relay 1 receives the Link Modification Ack message and forwards it to the Target UE since the Relay reselection indication is specified. The Link Modification Ack message is used to synchronise the switch of traffic between the Source UE and Target UE, via the selected Relay (e.g. RID2), at the same time.

12.	From this point on, Source UE and Target UE switch the IP traffic to the new PC5 unicast link via Relay 2.

* * * End of Changes * * *