3GPP TSG-SA2 Meeting #154	S2-2210897

# Toulouse, France, Nov 14 - 18, 2022											 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0155 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5.2.X 5G ProSe UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds 5G ProSe UE-to-UE Relay Discovery based KI#1 conclusion in TR 23.700-33. In order to resolve the NOTE 2, it proposes to add an UE-to-UE Relay operation supported indication of Source UE/Target UE/U2U Relay in the Model A/B Discovery message. NOTE 2: Whether and how a Source UE and a Target UE indicate support of UE-to-UE Relay operation will be determined in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe UE-to-UE Relay Discovery is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay Discovery is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.X, 5.8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.2.X	5G ProSe UE-to-UE Relay Discovery

For 5G ProSe UE-to-UE Relay discovery, both Model A and Model B discovery are supported:

-	Model A discovery is supported.

-	Model B discovery is supported.

- 	Discovery integrated into PC5 unicast link establishment procedure is supported

The procedures for 5G ProSe UE-to-UE Relay discovery are defined in clause 6.x.x.

* * * Next Changes * * *

5.8.X	Identifiers for 5G ProSe UE-to-UE Relay Discovery

The 5G ProSe UE-to-UE Relay discovery message contains two sets of elements, i.e. direct discovery set(s) and a U2U discovery set.

-	The direct discovery set of elements can be part of the contents of 5G ProSe Direct Discovery message as defined in Rel-17. This includes for example the User Info ID of Source UE and Target UE.

-	The U2U discovery set contains the information to support the discovery of the UE-to-UE relay and extensions of the direct discovery. This includes for example Type of Discovery Message, RSC, User Info ID of the relay, etc.

-	5G ProSe UE-to-UE relay only modifies the U2U set of the elements, and forwards the end-to-end elements during the discovery procedures.

For the 5G ProSe UE-to-UE Relay Model A Discovery, the Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, RSC, list of User Info ID of Target UE, optional an UE-to-UE Relay operation supported indication of 5G ProSe UE-to-UE Relay are contained in the Announcement message.

For the 5G ProSe UE-to-UE Relay Model B Discovery between Source UE and 5G ProSe UE-to-UE Relay, the Type of Discovery Message, User Info ID of Source UE, RSC, and User Info ID of Target UE, optional an UE-to-UE Relay operation supported indication of Source UE are contained in the Solicitation message, and the Type of Discovery Message, User Info ID of 5G ProSe UE-to-UE Relay, RSC, and User Info ID of Target UE are contained in the Response message.

For the 5G ProSe UE-to-UE Relay Model B Discovery between 5G ProSe UE-to-UE Relay and Target UE, the Type of Discovery Message, User Info ID of Source UE, User Info ID of 5G ProSe UE-to-UE Relay, RSC, and User Info ID of Target UE are contained in the Solicitation message, and the Type of Discovery Message, RSC, User Info ID of Source UE, and User Info ID and Layer-2 ID of Target UE, optional an UE-to-UE Relay operation supported indication of Target UE are contained in the Response message.

* * * End of Changes * * *