3GPP TSG-WG SA2 Meeting #154 	S2-2210982

Toulouse, France, November 14 – 18, 2022	(revision of S2-2210688)

Source:	ZTE, NEC

Title:	Conclusion update on KI#1

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: this paper is to propose partial conclusion of key issue #1.

1. Introduction/Discussion

There are several Editor’s Note remaining in conclusion for KI#1. This paper propose how to address them.

  - What is the trigger of slice replacement

Currently it is agreed that the AMF uses local configuration (e.g. based on trigger from OAM) or based on PCF notification.

One issue is which PCF is used for the triggering. Since the AM PCF already provides the DNN replacement, it is proposed that the AM PCF can also trigger the slice replacement. The details of service operation can be determined during normative phase.

Regarding further trigger event, it is proposed that the SMF can also trigger the slice replacement based on local configuration or SM PCF notification. The main reason is that for the roaming case the VPLMN has no knowledge about the slice load in HPLMN therefore only the H-SMF can trigger the slice replacement..

  - How the AMF/SMF determines the alternative S-NSSAI

It was agreed that the AMF can determine the alternative S-NSSAI by interacting with the PCF and/or with the NSSF. It should be clarified that the PCF is AM PCF.

There is proposal that the AMF should retrieve the URSP rules from the UE policy to further check whether the alternative S-NSSAI is within the URSP rule. However it is not needed since the AMF has no idea about the Traffic Descriptor trigger the PDU session.

The SMF can also determine the alternative S-NSSAI by interacting with the SM PCF, or NWDAF

  - Same S-NSSAI associated with multiple NSIs

This scenario is applicable for SMF triggering the slice remapping. The SMF reuses existing mechanisms of SSC mode2 and SSC mode 3 to trigger the UE to re-establish a new PDU session towards same S-NSSAI. During the new PDU session establishment, if the original NSI is overloaded the AMF is allowed to perform NSI re-selection based on local configuration.

  - Same NSI associated with multiple S-NSSAI

In this scenario, same mechanisms below can be reused to transfer the PDU session towards alternative S-NSSAI. There is no special handling needed for this scenario.

  - Different NSI associated with multiple S-NSSAI

1.5.1	AMF update the UE with alternative S-NSSAI

The AMF performs UE MM configuration update (e.g. UCU procedure) to notify the UE that the old S-NSSAI is now mapped to alternative S-NSSAI, so the UE does not establish new PDU session using the old S-NSSAI only. It is proposed that this update is sent after the PDU session transfer in order to avoid that old PDU session is released because it is not in the Allowed NSSAI. The alternative S-NSSAI is included in the Allowed NSSAI, and the old S-NSSAI is included in the mapping of the Allowed NSSAI.

For the roaming case, if the HPLMN S-NSSAI is changed triggered by H-SMF, the AMF does not update the UE. If the VPLMN S-NSSAI is to be replaced, the AMF update the UE with new mapping information of the old HPLMN S-NSSAI.

1.5.2	For a new PDU Session establishment request from the UE on the old S-NSSAI

This scenario is only for the UE without been updated by the AMF. It is proposed that the AMF continue the PDU session establishment with the alternative S-NSSAI. The PDU Session Establishment Accept message includes the alternative S-NSSAI so the UE knows the PDU session is now associated with the alternative S-NSSAI.

1.5.3	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

In order to serve service continuity, different SSC mode needs to be taken into account.

For SSC mode 1, the PDU session needs to be retained.

- AMF triggering

  - If the AMF determines that the SMF support the alternative S-NSSAI, the AMF updates the SMF with the alternative S-NSSAI. The SMF notifies the RAN and the UE about the alternative S-NSSAI.

  - If the AMF determines that the SMF does not support the alternative S-NSSAI, the AMF may insert/replace I-SMF/V-SMF supporting the alternative S-NSSAI. The new I-SMF/V-SMF notifies the RAN and UE about the alternative S-NSSAI.

  - The AMF update the UE in MM NAS with the alternative S-NSSAI.

- SMF triggering

  - If the SMF determines that it can serve the alternative S-NSSAI via resource repartition, it then notifies the RAN and UE about the alternative S-NSSAI

  - If the SMF determines that it cannot serve the alternative S-NSSAI, the SMF triggers the AMF to insert an I-SMF supporting the alternative S-NSSAI.

For SSC mode 2/3,

  - For both AMF triggering/SMF triggering, the SMF notifies the UE to re-establish a new PDU session with both old S-NSSAI and alternative S-NSSAI.

  - The UE establishes UE PDU session with both old S-NSSAI and alternative S-NSSAI

2. Text Proposal

Based on the analysis above, it is proposed to capture the following changes vs. TR 23.700-41 110.

# * * * * First change * * * *

8	Conclusions

8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE 0:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger in the AMF to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on AM PCF/NSSF notification. The UE capability to support this feature is a prerequisite for the execution of the optimizations in this conclusions.

NOTE 1:	How the SBI service used for AM-PCF notification will be determined in normative phase.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the AM PCF and/or with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

NOTE 2:	How the SBI service used for AM-PCF notification will be determined in normative phase.

2)	In case of a same S-NSSAI associated with multiple NSIs and a NSI can no longer be used, the NSI is no longer considered for binding S-NSSAI at the AMF and NSSF and when  a PDU session establishment request from the UE arrives at the AMF, the AMF and NSSF performs NSI re-selection without considering the old NSI as candidate NSI and the AMF selects another NSI

3)	In cases where the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, the mechanisms in bullet 4) is reused to transfer the PDU session to alternative S-NSSAI.

4)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that  the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and  independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

NOTE 3: 	Whether to use existing Mapping Of Allowed NSSAI or use an alternative IE is to be determined at normative phase.

-

b)	For a new PDU Session establishment request from the UE only on the old S-NSSAI(for UEs without existing PDU session and not updated with alternative S-NSSAI in step 4a):

-	The AMF proceeds with the PDU Session establishment to the SMF, and the AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI. The SMF sends the alternative S-NSSAI in the PDU Session Establishment Accept message so the UE knows the PDU session is associated with both the alternative S-NSSAI and old S-NSSAI.

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI, in order to ensure service continuity:

-	The AMF update the UE configuration according 4a).

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and indicates the alternative S-NSSAI.

-	The SMF may determine that the PDU session needs to be retained. In this case the SMF may send the alternative S-NSSAI to the UE in the PDU Session Modification Command, to the UPF in the N4 message and to the RAN in N2 message.

-	The SMF may determine that the PDU session needs to be re-established, In this case the SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates the alternative S-NSSAI to the UE to trigger the re-establishment of  the PDU Session  on the alternative S-NSSAI.

-	the UE triggers a new PDU Session Establishment procedure to establish a PDU Session as follows:

-	The PDU Session establishment request includes the alternative S-NSSAI together with the old S-NSSAI (similar as to use the mapped S-NSSAI value of the VPLMN when applying the RSD matched to the HPLMN S-NSSAI).

-

# * * * * End of changes * * * *