GPP TSG-SA WG2 Meeting #154	S2-2210990

# Nov 14 – 18, 2022, Toulouse																		     (revison of S2-2210266)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0237 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple location report for MT-LR Immediate Location Request for the regulatory service |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, T-Mobile USA, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MLR |  |  |  |  |  |  | Date: |  | 2022-11-14 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | In the case of regulatory services (e.g. emergency and lawful interception) it would be beneficial to receive multiple locations estimates in a single session for a target UE, this would enable the regulatory service to get more UE location information. - The Proposed solution introduces new procedures for MT-LR Immediate Location Request for the regulatory service to provide multiple location estimates to LCS Client for target UE during a short period of time. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | LMF in 5GC-MT-LR procedure for regulatory location service optionally can send more location estimated to GMLC by reusing Nlmf_location_EventNotify service operation. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If more location estimates are required than the more complex periodic deferred location must be used even for regulatory services. Existing periodic deferred location methods are designed for cLBS. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.1a.4, 6.1.x (new), 6.10.x (new) |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

* * * * Next Change * * * *

### 4.1a.4	Immediate Location Request

With an immediate location request, an LCS client or AF sends or instigates a location request for a target UE (or group of target UEs) and expects to receive a response containing location information for the target UE (or group of target UEs) within a short time period which may be specified using QoS. In regulatory cases, one or more responses of the target UE’s location information can be expected. An immediate location request may be used for an NI-LR, MT-LR or MO-LR.

* * * * Next Change * * * *

### 6.1.x	5GC-MT-LR multiple location procedure for the regulatory location service

Figure 6.1.x-1 illustrates an extension procedure of 5GC-MT-LR procedure for the regulatory location service defined in Clause 6.1.1.

This procedure is applicable for providing multiple location estimates of the target UE to LCS client.

Figure 6.1.x-1: 5GC-MT-LR multiple location procedure for the regulatory location service

- Steps 1-7 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1 are performed with the following differences:

- At step 1 in clause 6.1.1 the request from external location services client may include acceptance of INTERMEDIATE response and maximum response time.

- At step 4 in clause 6.1.1 Namf_Location_ProvidePositioningInfo service operation invoked by GMLC may include the acceptance of INTERMEDIATE response and maximum response time, GMLC contact address and LIR reference number. The LIR reference number is allocated by GMLC based on predefined rule.

- At step 7 in clause 6.1.1 Nlmf_Location_DetermineLocation service operation invoked by AMF may include additionally include acceptance of INTERMEDIATE response and maximum response time, GMLC contact address, LIR reference number for the INTERMEDIATE response. AMF can overwrite maximum response time to avoid timeout.

- LMF performs positioning procedures and determines multiple location estimates during the session.

  - This step is executed if the INTERMEDIATE result is available. The LMF invokes an Nlmf_Location_EventNotify service operation towards GMLC and provides the INTERMEDIATE location of the UE during the session and timestamp of the location estimate, and LIR reference number, and INTERMEDIATE event type.

  - This step is executed if step 2.1 was executed. The GMLC sends the INTERMEDIATE location of the UE and timestamp of the location estimate to external location services client.

- Based on Step 9 in clause 6.1.1 the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the FINAL location of the UE. LMF ensures that FINAL location will be transferred towards AMF within the maximum response time.

- Steps 10-11 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1 are performed with the following differences:

- At step 11 the FINAL location is sent from GMLC to LCS client.

* * * * Next Change * * * *

### 6.10.X	5GC-MT-LR multiple location procedure without UDM Query

Figure 6.10.x-1 illustrates an extension procedure of 5GC-MT-LR procedure without UDM Query for the regulatory location service defined in Clause 6.10.2.

This procedure is applicable for providing multiple location estimates of the target UE to emergency services client.

Figure 6.10.x-1: 5GC-MT-LR Multiple Location Procedure without UDM Query

- Steps 1-3 for 5GC-MT-LR procedure without UDM Query in clause 6.10.2 are performed with the following differences:

- At step 1 in clause 6.10.2 the request from external emergency client (e.g. a PSAP) location services client may include the acceptance of INTERMEDIATE response and maximum response time.

- At step 2 in clause 6.10.2 Namf_Location_ProvidePositioningInfo service operation invoked by GMLC may include the acceptance of INTERMEDIATE response and maximum response time, GMLC contact address and LIR reference number. The LIR reference number is allocated by GMLC based on predefined rule.

- At step 3 in clause 6.10.2 Nlmf_Location_DetermineLocation service operation invoked by AMF may include the acceptance of INTERMEDIATE response and maximum response time, GMLC contact address, and LIR reference number. AMF can overwrite maximum response time to avoid timeout.

- LMF performs positioning procedures and determines multiple location estimates during the session.

  - This step is executed if INTERMEDIATE response is available. The LMF invokes an Nlmf_Location_EventNotify service operation towards GMLC and provides the INTERMEDIATE location of the UE and timestamp of the location estimate, and LIR reference number, and INTERMEDIATE event type.

  - This step is executed if step 2.1 was executed. The GMLC sends the INTERMEDIATE location of the UE and timestamp of the location estimate to external location services client.

- Based on Step 5 in clause 6.10.2 the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the FINAL location of the UE. LMF ensures that FINAL location will be transferred towards AMF within the maximum response time.

- Steps 6-7 for 5GC-MT-LR procedure without UDM Query in clause 6.10.2 are performed with the following differences:

- At step 7 the FINAL location is sent from GMLC to the external services client.

* * * * End of Changes * * * *