3GPP TSG-SA WG2 Meeting #154	S2-2211068

14 - 18 November, 2022, Toulouse, France	(revision of S2-2210662)

# Title:	Reply LS on UE Policy Control with PCF re-selection during AMF relocation

# Response to:	(S2-2210170/C3-224697) UE Policy Control with PCF re-selection during AMF relocation

# Release:	Release 15/16/17

# Work Item:	5GS_Ph1

Source:	SA2

To:	CT3

Cc:	CT1

Contact Person:

Name:	Pallab Gupta

Tel. Number:

E-mail Address:	pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: TS 23.503 CR 0786, TS 23.502 CR 3635.

1. Overall Description:

SA2 thanks CT3 for their LS on UE Policy Control with PCF re-selection during AMF relocation.

SA2 has discussed the questions and would like CT3 to consider the answer provided below:

Question 1: At the reception of UE Policy Association creation request, when no indication about the list of stored PSIs, ANDSP support, and UE OSId is received, shall the PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored? If not, how does the PCF determine it is a different scenario?

Answer 1:  SA2 could not reach an agreement on whether the PCF needs to differentiate the 2 scenarios or whether the existing procedure is sufficient. SA2 is further seeking feedback from CT1 (via LS S2-2211072) on UE behaviour when UE PSIs are delivered to the UE in order to decide the way forward. SA2 would provide definitive feedback to CT3 after consulting with CT1.

Question 2: Additionally, since the new selected PCF does not receive information about the UE's support for ANDSP or UE’s support of OSIds, which affect to the proper determination of the ANDSP,

how does the ANDSP policies procedure work in this scenario (considering also roaming and V-PCF determination of ANDSP)?

Answer 2: As described in clause 6.1.2.2.2 of TS 23.503, The PCF stores the UE's support for ANDSP or UE’s support of OSIds in the UDR using the Nudr_DM_Update including DataSet "Policy Data" and Data Subset "UE context policy control data". For the non-roaming scenario, the new selected PCF can retrieve them from the UDR. For the roaming scenario, the H-PCF forwards the information to the V-PCF if the H-PCF has retrieved the information from the H-UDR. This is clarified in the attached CR to TS 23.502 and TS 23.503.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154AH	 Jan 16 - 20, 2023				Elbonia

TSG-SA2 Meeting #155	Feb 20 - 24, 2023				Athens, Greece