3GPP TSG-WG SA2 Meeting #154 	S2-2211124

Toulouse, France, November 14 – 18, 2022	(revision of S2-2210501)

Source:	Huawei, HiSilicon, Apple?, CATT?, Cisco?, Ericsson?, Futurewei?, Google Inc.?, Intel?, Lenovo?, MediaTek Inc.?, Meta USA?, Nokia?, Nokia Shanghai Bell?, Qualcomm Incorporated?,Samsung, Sony?, Tencent?, Tencent Cloud?, OPPO?, vivo?, Xiaomi?, ZTE?

Title:	Conclusion update for KI#4&5

Document for:	Approval

Agenda Item:	9.19.1

Work Item / Release:	FS_XRM / Rel-18

Abstract: Conclusion update for KI#4&5 to resolve the ENs.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 8.4	Conclusions for KI#4 and KI#5

The following aspects are concluded as principles for the normative work to support the following two key issues:

-	Key Issue #4: PDU Set integrated packet handling

-	Key Issue #5: Differentiated PDU Set Handling

NOTE 1:	Further PDU Set handling for Uplink will be studied and led by RAN WG. SA2 can align with RAN’s progress and decision for Uplink, if any.

NOTE 2:	If the DL PDU Set eligible dropping by the NG-RAN applied, the charging offset issue can be addressed during the normative work.

### 8.4.1	Control plane enhancements for supporting PDU Set in downlink

#### 8.4.1.1	PDU Set QoS Parameters

PDU Set QoS treatment is determined using dynamic or non-dynamic PCC.

The following PDU Set QoS parameters are defined to support PDU Set handling:

NOTE 1: The definitions of PSER and PSDB can be revisited during normative phase.

-  PDU Set Error Rate: The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PSER is the same in UL and DL. If any PDU within the PDU Set is not successfully transmitted, the PDU Set is treated as error.

NOTE 2: In this release, it assumes all PDUs of a PDU Set are needed for application to use the PDU Set.

-  PDU Set Delay Budget: The PSDB defines an upper bound for the time that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

NOTE 3: To enable support for PSDB, it is assumed that there is a maximum duration threshold for inter arrival time between PDUs and first arrived PDU within the PDU Set as per SLA or pre-configuration.

PSDB is an optional parameter that applies only when the maximum duration threshold is met. Otherwise, PSDB is not applied and PDB should be applied instead. And the NG-RAN can still take into account the PDU Set concept for scheduling (the detail is discussed in normative phase).

If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.4.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN.

NOTE 4: Whether to define new 5QI(s) related to PSDB/PSER for supporting XRM services can be determined during the normative phase.

#### 8.4.1.2	AF Information Provisioning

PDU Set related assistance information provisioning by AF is supported for dynamic PCC. AF may provision one or more of the following PDU Set related assistance information to NEF/PCF during AF QoS request procedure:

-  PDU Set QoS parameters listed in clause 8.4.1.1.

-  Burst periodicity

-	Description of Service Protocol: Indicates RTP/SRTP header type to be used for PDU Set Identification at UPF.

### 8.4.2	User plane enhancements for supporting PDU Set in downlink

#### 8.4.2.1	PDU Set Information

The following PDU Set related information are identified by UPF to support PDU Set based handling:

-  PDU Set Sequence Number

-  Optional, End PDU of the PDU Set

-  Optional, PDU SN within a PDU Set

-  Optional, PDU Set Size in bytes.

NOTE 1: If PDU set size is known by UPF, it should be sent to RAN.

-	PDU Set Importance: This parameter is used to identifies the importance of a PDU Set within a QoS flow. RAN may use it for PDU Set level packet discarding in presence of congestion.

#### 8.4.2.2	PDU Set Information identification on UPF and supported N6 protocols

The detection and marking of the DL PDU Sets sent to the NG-RAN shall be done by the PSA UPF.

PSA UPF may identify the PDU Set based on instruction from SMF and packet header of N6 protocols:

-  by matching RTP/SRTP header and payload (RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking are supported).

NOTE 1: In above cases, it is assumed that the RTP/SRTP header and/or payload necessary for the identification of PDU Set Information is not encrypted.

NOTE 2: Support of new RTP header extension for PDU Set identification depends on progress in SA4 5G_RTP WI.

-  by UPF implementation, e.g., PDU Set detection based on traffic characteristics. IP header parameters DSCP/TOS, IP port, IPv6 flow label may be used to detect PDU set, however detailed mechanisms in UPF for PDU Set information identification will not be standardized.

#### 8.4.2.3	Delivering PDU Set Information to RAN

PDU Set Information (listed in 8.4.2.1) (except for PDU Set Importance) are informed by UPF to RAN via GTP-U header of user plane packet.

Working assumption: For PDU Set Importance, the PSA UPF marks PDU Set Importance on the GTP-U header of DL packets of a PDU Set.

### 8.4.3	PDU Set based QoS handling

RAN performs PDU Set based QoS handling based on received PDU Set QoS Parameters via control plane, and PDU Set Information received via user plane. The details of RAN behaviours are defined in RAN WG.

# * * * * End of changes * * * *