3GPP SA WG2 Meeting #154	S2-2211175

14 - 18 November, 2022, Toulouse, France	(revision of S2-2210852)

Source: 	Intel, Google Inc., Cisco, Apple, Meta USA

Title: 	KI#4, KI#5: Addition of missing codec in the conclusion

Document for: 	Approval

Agenda Item: 	9.19.1

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes to add AV1 in the list of codecs for which the PSA UPF may identify the PDU Set based on matching RTP/SRTP header and payload.

# 1	Discussion

The AV1 codec is currently missing from the RTP/SRTP header and payload options in the agreed conclusions in clause 8.4.2.2:

-	by matching RTP/SRTP header and payload (RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking are supported).

In our understanding this is a simple omission, given that AV1 is referenced in candidate solutions #17 and #18 in TR 23.700-60. Also, the LS to SA4 sent out from SA2#153E (S2-2209905) explicitly references AV1, as in the following excerpt:

Option 2. Define new protocol (e.g., RTP/SRTP) header extensions by taking into account Network Abstraction Layer (NAL) units, RTP Payload type (e.g., H.264/5/6 and VP9/AV1), etc., to identify PDU Sets in DL, including, e.g., PDU set sequence number, PDU Set size in bits, PDU Set length in number of PDUs, PDU sequence number within the PDU set.

Based on this, it is proposed to add AV1 in the list of codecs for which the PSA UPF may identify the PDU Set based on matching RTP/SRTP header and payload.

# 2	Proposal

It is proposed to agree the proposed text for inclusion in TR 23.700-60.

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]	IEEE SA P1918.1: "Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions", https://standards.ieee.org/project/1918_1.html.

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[9]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[10]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[11]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

[12]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

[13]	IETF RFC 6190: "RTP Payload Format for Scalable Video Coding", May 2011.

[14]	IETF RFC 6437: "IPv6 Flow Label Specification", November 2011.

[15]	IETF RFC 8285: A General Mechanism for RTP Header Extensions.

[16]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[17]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[18]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[19]	ITU-T Recommendation H.264: "Advanced video coding for generic audiovisual services".

[20]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control", July 2003.

[21]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)", March 2016.

[22]	ISO/IEC 14496-12 | 15444-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format".

[23]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[24]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[25]	IETF RFC 1889: "RTP: A Transport Protocol for Real-Time Applications", January 1996.

[26]	3GPP TS 29.998: "Open Services Architecture (OSA) Application Programming Interface (API) part 2".

[27]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[28]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[29]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[30]	IETF RFC 5888 (2010): "The Session Description Protocol (SDP) Grouping Framework", G. Camarillo, H. Schulzrinne.

[31]	IETF RFC 3261 (2002): "SIP: Session Initiation Protocol", J. Rosenberg et al.

[32]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[33]	IETF RFC 3665: "Session Initiation Protocol (SIP) Basic Call Flow Examples", A. Johnston et al.

[34]	WebRTC TR: https://www.w3.org/TR/webrtc/.

[35]	IETF RFC 7272: "Inter-Destination Media Synchronization (IDMS) Using the RTP Control Protocol (RTCP)".

[36]	IETF RFC 8311: Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation.

[37]	IETF Internet-draft - L4S: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture", Briscoe et al, Nov 2020, https://datatracker.ietf.org/doc/draft-ietf-tsvwg-l4s-arch/.

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[38]	IETF Internet draft: "Explicit Congestion Notification (ECN) Protocol for Ultra-Low Queuing Delay (L4S)", Briscoe et. al, Internet draft, IETF, Mar 2021, https://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-ecn-l4s-id/.

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[39]	EU project RITE: "Data Center to the Home", https://riteproject.eu/dctth/.

[40]	L4S: "Linux Kernel tree containing patches for TCP Prague and the dualpi2 qdisc", https://github.com/L4STeam/linux/.

[41]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1", June 1999.

[42]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)", May 2015.

[43]	IETF draft-ietf-quic-http-34: "Hypertext Transfer Protocol Version 3 (HTTP/3)", February 2021.

[44]	IETF RFC 6994: "Shared Use of Experimental TCP Options", August 2013.

[45]	IETF draft-ietf-tsvwg-udp-options-18: "Transport Options for UDP", March 2022.

[46]	ITU-T Recommendation H.265: "High efficiency video coding".

[47]	ITU-T Recommendation H.266: "Versatile video coding.

[48]	draft-ietf-avtcore-rtp-vvc-14: "RTP Payload Format for Versatile Video Coding (VVC)".

[49]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[50]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[51]	IETF RFC 9001: "Using TLS to Secure QUIC".

[52]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[53]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[54]	draft-ietf-masque-connect-udp: "UDP Proxying Support for HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[55]	draft-ietf-quic-http: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[56]	draft-ietf-masque-h3-datagram: "Using Datagrams with HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[57]	draft-ietf-httpbis-h3-websockets: "Bootstrapping WebSockets with HTTP/3".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[58]	3GPP TR 38.838: "Study on XR (Extended Reality) evaluations for NR".

[59]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[60]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[61]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[62]	IETF Internet-draft-L4S: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)" Briscoe et al, July 2022, https://datatracker.ietf.org/doc/draft-ietf-tsvwg-ecn-l4s-id/.

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[63]	IETF draft-gruessing-moq-requirements-02: "Media over QUIC – Use Cases and Requirements for Media Transport Protocol Design".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[64]	IETF draft-lcurley-warp-01: "Warp – Segmented Live Media Transport".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[65]	draft-schinazi-masque-connect-udp-ecn-02: "An ECN Extension to CONNECT-UDP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[66]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[67]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

*** NEXT CHANGE ***

# 8	Conclusions

#### 8.4.2.2	PDU Set Information identification on UPF and supported N6 protocols

The detection and marking of the DL PDU Sets sent to the NG-RAN shall be done by the PSA UPF.

PSA UPF may identify the PDU Set based on instruction from SMF and packet header of N6 protocols:

-	by matching RTP/SRTP header and payload.

Editor's note:	Whether support PDU Set identification information in new RTP is pending to SA WG4 5G_RTP WI.

NOTE:	In above cases, it is assumed that the RTP/SRTP header and/or payload necessary for the identification of PDU Set Information is not encrypted.

-	by UPF implementation, e.g. PDU Set detection based on traffic characteristics. IP header parameters DSCP/TOS, IP port, IPv6 flow label may be used to detect PDU set, however detailed mechanisms in UPF for PDU Set information identification will not be standardized.

Editor's note:	Other N6 protocols, i.e. HTTP/MASQUE, GTP-U, IP/TCP/UDP/QUIC options, carrying PDU Set information are FFS. (Potential SoH).

*** END CHANGES ***