3GPP TSG-SA WG2 Meeting #154 	S2-2211258

# Toulouse, France, November 14 – 18, 2022	(revision of S2-2211160)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0584 | rev | 3 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF-assisted application detection in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Ericsson, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on conclusion part for KI#2 in clause 8.2 of TR 23.700-81, it is concluded to focus on the use case of PFD updates for known applications in Solution#9, which is selected as baseline for the normative work. Therefore, this proposal specifies the procedure of NWDAF-assisted PFD determination in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the detailed procedure of NWDAF-assisted PFD determination. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for NWDAF-assisted PFD determination will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (All new text)

6.X	PFD Determination Analytics

6.X.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics, to analytics

To assist on PFD determination for known application identifiers, if the related Service Level Agreement does not preclude, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new/updated PFDs to analytic consumer in the 5GC. The NEF(PFDF) as the consumer may forward new/updated PFD information provided by the NWDAF to UPF via SMF to detect a known application defined as TS 23.502 [3].

The service consumer may be a NEF(PFDF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	Application ID list;

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new/updated PFDs).

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.X.2	Input Data

NWDAF collects QoS flow related data from NF(s) for a specific S-NSSAI, DNN, and UE(s). The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that uses the application |
| S-NSSAI | SMF, UPF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF, UPF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Application ID | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain name matching criteria and information about applicable protocol(s). |

NOTE 1:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.X.3-1. The output analysis can be used to provision new/updated PFDs for known applications.

Table 6.X.3-1: PFD Determination  statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application description analytics |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| > URL | the significant parts of the URL to be matched, e.g. host name defined in. |
| > Domain name information | A Domain name matching criteria and information about applicable protocol(s). |

6.X.4	Procedures

The procedure depicted in 6.X.4-1 shows the procedure that the NWDAF can provide PFD Detection analytics to a  NEF (PFDF)).

Figure 6.X.4-1: A procedure for NWDAF-assisted PFD Determination

The Consumer NF (NEF(PFDF)) requests or subscribes to the NWDAF to request PFD determination analytics. The Target of Analytics Reporting is set to Any UE. A single App-ID or a list of App-IDs are also provided as input. This subscription may be triggered by local configuration or OAM. The Analytics Filter Information may optionally include the S-NSSAI and/or DNN2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF).

3.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets corresponding to Application ID from the SDF not matching installed PDRs.

Editor’s Note: UPEAS WI is specifying UPF event exposure for NWDAF data collection. It is FFS whether TS 23.502 clause 4.15.and the other UPEAS normative work address the needs of this new analytic.

4.	The NWDAF derives PFD determination analytics, e.g., new/updated PFD information for the existing Application ID.

5.	The NWDAF notifies PFD determination analytics to the Consumer NF (NEF(PFDF)) with PFD Information.

END of CHANGES