3GPP TSG-WG SA2 Meeting #154	S2-2211288

# Toulouse, France, November 14 – 18, 2022		(revision of S2-2211273, 0954, 0616)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0144 | rev | 3 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe UE-to-UE Relay reference architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds 5G ProSe UE-to-UE Relay reference architecture based on the KI#1 conclusion in TR 23.700-33. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe UE-to-UE Relay reference architecture is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay reference architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.X	5G ProSe UE-to-UE Relay reference architecture

Figure 4.2.X-1 shows the Layer-2 and Layer-3 5G ProSe UE-to-UE Relay reference architecture. The 5G ProSe End UEs communicate with each other via a 5G ProSe UE-to-UE Relay.

Figure 4.2.X-1: Reference architecture for 5G ProSe UE-to-UE Relay

Each 5G ProSe End UE and the 5G ProSe UE-to-UE Relay may have subscriptions from the same PLMN or different PLMNs.

# * * * * End of changes * * * *