3GPP TSG-WG SA2 Meeting #154 	S2-2211459_rm

Toulouse, France, November 14 – 18, 2022	(e-mail revision of S2-2210308r03)

(Revision of SP-220989)

Source:	Huawei, HiSilicon, Samsung

Title:	Revised WID on generic group management, exposure and communication enhancements

Document for:	Approval

Agenda Item:	10.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Generic group management, exposure and communication enhancements

Acronym:	GMEC

Unique identifier:	970069

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_GMEC | SA2 | 940061 | Study on generic group management, exposure and communication enhancements |
| FS_5GLAN_enh | SA2 | 840025 | Study on enhancement of support for 5G LAN-type service |
| Vertical_LAN | SA2 | 830042 | 5GS Enhanced support of Vertical and LAN Services |
| SEI | SA1 | 920039 | Smart Energy and Infrastructure |
| QoS_MON | SA1 | 790004 | QoS Monitoring |
| cyberCAV | SA1 | 800007 | Service requirements for cyber-physical control applications in vertical domains |
| eCAV | SA1 | 840050 | Enhancements for cyber-physical control applications in vertical domains |
| EXPOSE | SA1 | 930023 | Service exposure interfaces for industry |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

3GPP has specified many enablers for vertical use, for example the features defined by IIOT, NPN and URLLC. Among them, R16 5G LAN-type service offers private communication using IP and/or non-IP type communications for UE and devices behind UE, using an optimized communication path controlled by a single SMF. However, the cardinality of SMF per 5G VN is limited to one in Rel-16, even if the 5G VN is very large. Multiple SMFs should be supported, for instance for administrative domains or a large multi-site company or a large scale industrial setting that spans multiple countries. And at the same time, the SMFs need to support single/common 5G VN wide area.

In Rel-18 SA1 study on 5G Smart Energy and Infrastructure “5SEI” has concluded new requirement (clause 6.13.2, 6.28 of TS 22.261 and clause 5.2, 5.6 and 9 of TS 22.104) including that the 5G system shall allow a UE to request a communication service to send data to different groups of UEs at the same time and the 5G system shall allow different QoS policy for each group the UE communicates with.

Recently, the 5G Alliance for Connected Industries and Automation (5G-ACIA) has provided to 3GPP a whitepaper (S2-2102128) including a set of functional requirements that the 5GS has to satisfy in terms of supporting certain information exchange between 5GC and industrial application domain, and exposure of 5G capabilities. The main goal is to enable the management, operation, monitoring and use of such networks and network services from an enterprise perspective easily without having to rely on sophisticated, heavy-weight tools and in-depth knowledge on the underlying 5G technology. Some requirements regarding device management, e.g. connectivity management, connectivity monitoring, group management etc. have not yet been fulfilled and need further studies.

The study on generic group management, exposure and communication enhancements completes the evaluations and conclusions on KI#1, KI#2, KI#3 and KI#5, thus the corresponding Work Item is required in order to continue specifying the support of those concluded key issues in the normative work.

# 4	Objective

This Work Item implements the conclusions of the FS_GMEC Study Item in TR 23.700-74, as follows:

-	KI#1: Enhance group attribute management for groups created by an AF using the 5G VN API as well as groups other than 5G VN groups configured by O&M as in clause 8.1 of TR 23.700-74, including:

-	support provisioning of service area applicable to each UE within the group

-	support of LADN per DNN and S-NSSAI for enforcement of service area applicable to each UE within the group

-	support provisioning and enforcement of default QoS applicable to each UE within the group

-	support AF indication that the group is associated with 5G VN group communication

-	support of Group-MBR for a 5G VN group using Slice-MBR (PCF/UDR based method)

-	KI#2: Enhance group status event reporting for groups created by an AF using the 5G VN API as well as groups other than 5G VN groups configured by O&M as in clause 8.2 of TR 23.700-74

-	support reporting for group member list changes via event exposure service

-	KI#3: NEF exposure framework for provisioning of traffic characteristics and monitoring of performance characteristics for a group of UEs as in clause 8.3 of TR 23.700-74, including:

-	support mapping the request targeting a group to requests targeting each group member within the group

- 	handle request for traffic characteristics based on 5G QoS parameters

NOTE 1: 	The QoS and policy framework is re-used for parameter enforcement. The existing QoS Parameter Notification Control and existing AQP can be used for monitoring of specific performance characteristic. A NOTE can be added to clarify how AF handles request for UE-to-UE traffic i.e. initial "QoS division" and updated "QoS division".

-	handle AF-requested change of PDU Session Type associated with a group of UEs

-	manage temporal invalidity/validity condition (start-time, end-time)

-	KI#5: Allowing UE to simultaneously send data to different groups with different QoS policy as in clause 8.5 of TR 23.700-74. This has no impacts to UE, SMF and UPF.

-	use different QoS Flows of single PDU Session or different PDU Sessions to transfer the data copy sent to different groups

-	or, associate each group and group combination with separate multicast address to allow application to send data to different groups using the multicast address associated with the group(s) it wants to target

NOTE 2:	No normative changes will result from the two sub-bullets above.

-	support provisioning of QoS applicable to service flow corresponding to a multicast address for each group member within the group using outcome of KI#3

# 5	Expected Output and Time scale

| Impacted existing TS |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | CRs for implementation of KI#1, 2, 3, 5 |  |  |
| TS 23.502 | CRs for implementation of KI#1, 2, 3, 5 |  |  |
| TS 23.503 | CRs for implementation of KI#1, 2, 3, 5 |  |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur, Qianghua Zhu, Huawei, zhuqianghua@huawei.com (for work planning and KI#1, 2, 3)

Secondary Rapporteur, Sang-Jun Moon, Samsung, moonst@samsung.com (for KI#5)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

SA3 for security aspects. SA5 for management and charging aspects. SA6 for application layer aspects.

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Samsung

CATT

China Mobile

China Telecom

China Unicom

ZTE

KT Corp

LG Uplus

LGE

ETRI

CAICT

CEPRI

CBN

SIA

EDF

DISH

NTT Docomo

Futurewei

MATRIXX Software