# 3GPP SA WG2 Meeting #154	S2-2210684r04

# Toulouse, France, November 14 – 18, 2022	(Revision of S2-220xxxx)

# Source:	ZTE, LG Electronics

# Title:	New WID: Enhancement of Network Slicing Phase 3

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Stage 2 of Network Slicing Phase 3

## Acronym: eNS_Ph3

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x | x |  |
| No |  |  |  |  |  |
| Don't know | x |  |  |  | x |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_eNS_Ph3 | SA2 | 940063 | Study on Network Slicing Phase 3 |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 910032 | Enhanced Access to and Support of Network Slice | Stage 1 requirements |

## 3	Justification

This work item aims at specifying system enhancements on network slicing as concluded within TR 23.700-41.

## 4	Objective

The objective is to specify the following enhancements to 5GS as per conclusions reached within TR 23.700-41 for the following aspects:

For key issue #1

-	Support the AMF determining alternative S-NSSAI and triggering the S-NSSAI replacement by interaction with AM PCF / NSSF.

-	Allow the AMF in early binding to perform NSI re-selection when the UE establish a new PDU session.

-	Enhance the system to provide alternative S-NSSAI to the supporting UE via Registration Procedure or UE Configuration Update Procedure.

-	Support S-NSSAI replacement for a new PDU Session by supporting the SMF to provide alternative S-NSSAI to UE upon receiving alternative S-NSSAI from AMF.

-	Support S-NSSAI replacement for an existing PDU Session without PDU session re-establishment by supporting the SMF to provide alternative S-NSSAI to UE, RAN, and UPF upon receiving alternative S-NSSAI from AMF.

-	Support S-NSSAI replacement for an existing PDU Session using PDU session re-establishment by supporting the SMF to provide alternative S-NSSAI to UE upon receiving alternative S-NSSAI from AMF.

For key issue #2

-	Support of slice based SoR mechanism by referencing to TS 23.122.

For key issue #3

-	Define the network slice validity policy

-	Enhance the system to provide network slice validity policy to the UE via Registration Procedure or UE Configuration Update Procedure.

-	Support of slice decommissioning for supporting and non-supporting UEs.

For key issue #4

-	Enhance the Number of UEs per network slice availability check and update procedure to convey the Service Area and PLMN ID to NSACF

-	Define a Primary NSACF to handle overall NSAC for an S-NSSAI at the global level, and enhance the NSACF to support interaction with Primary NSACF for UE admission quota-based control, or UE admission threshold based control.

-	Define a Centralized architecture with a single centralized NSAC node to handle a PLMN with multiple service areas.

-	Support of NSAC (HPLMN control, VPLMN control and HPLMN delegated control) for roaming UE

For key issue #5

-	Enhance the system to provide information related to the supported or not supported TA(s) regarding a rejected S-NSSAI to the UE via Registration Procedure or UE Configuration Update Procedure.

-	Enhance the system to provide information related to the supported or not supported TA(s) regarding some S-NSSAIs which are partially/conditionally allowed in the RA to the UE via Registration procedure or UE Configuration Update procedure.

For key issue #6

-	Define the slice specific policy related to S-NSSAIs subject to network control and the network control the S-NSSAI deregistration and PDU session release

-	Enhance the system to configure and handle the slice specific policy in PCF or local AMF configuration

-	Support transfer of all PDU sessions from one S-NSSAI to a replacement S-NSSAI leveraging the procedures of Key issue # 1.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A | N/A | N/A | N/A | N/A | N/A |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Architectural enhancements to support the stated objectives |  |  |
| 23.502 | Procedural enhancements to support the stated objectives |  |  |
| 23.503 | Procedural enhancements to support the stated objectives |  |  |

## 6	Work item Rapporteur(s)

ZHU Jinguo, ZTE, zhu.jinguo@zte.com.cn, Primary Rapporteur (KI#1, KI#3, KI#4)

Myungjune Youn, LG Electronics, m.youn@lge.com, Secondary Rapporteur (KI#2, KI#5, KI#6)

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA5 for management and charging aspects.

SA3 for security aspects

## 9	Supporting Individual Members

Supporting IM name

Alibaba ?

Apple ?

AT&T ?

CATT ?

China Mobile ?

China Telecom ?

China Unicom ?

Convida Wireless LLC ?

Deutsche Telekom AG ?

DISH Network ?

Ericsson ?

Huawei ?

Intel ?

InterDigital ?

KDDI

KPN ?

KT Corp. ?

Lenovo ?

LG Electronics

LG Uplus ?

MATRIXX Software

MITRE ?

Motorola Mobility ?

NEC ?

Nokia ?

Nokia Shanghai Bell ?

NTT Docomo ?

OPPO ?

Oracle ?

Orange ?

Qualcomm ?

Samsung

Sanechips ?

Sharp ?

SK Telecom ?

T-Mobile USA ?

Spreadtrum ?

Tencent ?

TELUS ?

Verizon UK Ltd ?

Xiaomi ?

ZTE