3GPP TSG-SA2 Meeting #154 	S2-2210970

# Toulouse, Nov 14 - Nov 18, 2022                                               (Revision of S2-2210335)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0563 | rev | R1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new paramters to Redundant Transmission Experience analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Futurewei, Huawei, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 23.288 has defined Analytics ID = "Redundant Transmission Experience" to be provided by NWDAF and consumed by other NFs such as SMF to assist URLLC services. As specified in 3GPP TS 23.288, this analytic ID may be used by SMF to determine whether redundant transmission shall be performed, or (if it had been activated) shall be stopped. However, it is not defined how network can decide which redundant transmission mechanism (e.g., end-to-end redundant user plane paths or support of redundant transmission on N3/N9 interfaces) is needed based on the existing output analytics, specifically based on the Redundant Transmission Experience value (statistics/predictions) which calculated based on end-to-end measurements as input data. To this end, this CR aims at specifying theses enhanced input and output paramters as concluded in TR 23.700-81 clause 8.6. In the conclusion, it is proposed that the PCF can update the URSP to the UE to establish the redundant PDU Session, taking Primary RAN Path Redundant Transmission Experience and Secondary (Redundant) RAN Path Redundant Transmission Experience into account to decide if the redundant transmission mechanism is needed or not for the corresponding application, as described in Sol#49. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.13.1 is updated with new analytics filtering information. Clause 6.13.2 is updated with new input data parameters. Clause 6.13.3 is updated with new output data parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing analytics paramters are not enough to assist SMF/PCF to decide which redundant transmission option is needed and conclusion of TR cannot be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13.1, 6.13.2, 6.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outecome of SI FS_eNA_ph3 KI#6 and WI code for the work is not assigned yet & will replace WI code DUMMY when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.13.1	General

This clause describes the Redundant Transmission Experience related analytics. These analytics may be used as follows:

- by the SMF to determine whether redundant transmission on N3/N9 interfaces (see clause 5.33.2.2 of TS 23.501 [2]) shall be performed, or (if it had been activated) shall be stopped.

- by the PCF to determine whether the redundant transmission on end to end User Plane Paths (see clause 5.33.2.1 of TS 23.501 [2]) shall be performed, or (if it had been activated) shall be stopped.

The service consumer may be a NF (e.g. SMF, PCF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Redundant Transmission Experience".

-	Target of Analytics Reporting: a single UE (SUPI), any UE, or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-    PDU Session ID

-	optional list of analytics subsets that are requested (i.e. UL/DL packet drop rate GTP-U, UL/DL packet delay GTP-U, see clause 6.13.3).

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of Redundant Transmission Experience:

-	ordering criterion: "time slot start"; and

-	order: ascending or descending;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

* * * * Next Change * * * *

### 6.13.2	Input Data

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to collect UE mobility information from OAM, 5GC and AFs and service data from AF, as described in clause 6.7.2.2. In addition, NWDAF shall be able to collect the information for PDU session which is established with redundant transmission from the SMF. UE mobility information is specified in Table 6.7.2.2-1 and service data from AF related to UE mobility in Table 6.7.2.2-2. In addition, the NWDAF shall be able to collect performance measurements on user data congestion as specified in Table 6.8.2-1 for user data congestion analytics.

Additionally, the NWDAF collects the following input (see Table 6.13.2-1 and Table 6.13.2-2) according to existing measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2].

Table 6.13.2-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet drop rate GTP-U | OAM (see NOTE 2) | UL/DL packet drop rate measurement on GTP path on N3. |
| UL/DL packet delay GTP | OAM (see NOTE 2) | UL/DL packet delay measurement round trip on GTP path on N3. |
| E2E UL/DL packet delay | UPF | End-to-End (between UE and UPF) packet delay measurements. |
| E2E UL/DL packet drop rate | OAM | End-to-End (between UE and UPF) packet drop rate measurements. |
| QoS monitoring indicator | SMF (see NOTE 3) | Indicator which shows SMF has sent QoS monitoring request. |
| UL/DL packet delay RAN | UPF/SMF (see NOTE 3, 4) | RAN part of UL/DL packet delay measurement. |
| NOTE 1: The information in this table is provided both as the base to compare with the redundant transmission performance as well as when redundant transmission is enabled. NOTE 2: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 1 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements. NOTE 3: As specified in clause 5.33.3 of TS 23.501 [2], the NG-RAN initiates the RAN part of UL/DL packet delay measurement based on the QoS Monitoring request from SMF. NG-RAN reports the RAN part of UL/DL packet delay result to the PSA UPF in the UL data packet or dummy UL packet. NOTE 4: It is assumed UL/DL packet delay RAN includes the measurements for both primary and secondary paths. As specified in clause 5.33.3 of TS 23.501 [2], if the redundant transmission on N3/N9 interfaces is activated, the UPF and NG-RAN performs QoS monitoring for both UP paths. The UPF reports the packet delay of the two UP paths respectively to the SMF. |  |  |

Table 6.13.2-2: The information related to PDU Session established

| Information | Source | Description |
| --- | --- | --- |
| DNN | SMF | Data Network Name associated for URLLC service. |
| UP with redundant transmission | SMF | Redundant transmission is setup or terminated. |

* * * * Next Change * * * *

### 6.13.3	Output Analytics

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to provide Redundant Transmission Experience analytics as defined in Table 6.13.3-1 and Table 6.13.3-2.

If a service consumer is SMF, the Redundant Transmission Experience analytics can be used to make decision if that redundant transmission on N3/N9 shall be performed or (if activated) shall be stopped regarding the PDU session for URLLC service. SMF may take into consideration of Primary N3 Tunnel Path Redundant Transmission Experience and Secondary N3 Tunnel Path Redundant Transmission Experience output paramters (calculated based on input data specifically based on E2E UL/DL packet delay and UL/DL packet delay RAN) to decide if redundant transmission on N3/N9 shall be activated/stopped.

If a service consumer is PCF, the Redundant Transmission Experience analytics can be used to make decision if E2E redundant transmission shall be performed or (if activated) shall be stopped regarding the PDU session for URLLC service. PCF may take into consideration of Primary RAN Path Redundant Transmission Experience and Secondary RAN Path Redundant Transmission Experience output paramters (calculated based on input data specifically based on E2E UL/DL packet delay and UL/DL packet delay RAN) to decide if E2E redundant transmission shall be activated/stopped.

Table 6.13.3-1: Redundant Transmission Experience statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE, any UE, or a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the Redundant Transmission Experience statistics applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| > Observed Redundant Transmission Experience | Observed Redundant Transmission Experience related information during the Analytics target period. |
| >> UL/DL packet drop rate GTP-U (NOTE 2) | Observed UL/DL packet drop rate on GTP-U path on N3 (average, variance). |
| >> UL/DL packet delay GTP-U (NOTE 2) | Observed UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |
| >> E2E UL/DL packet drop rate | Observed End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |
| >> E2E UL/DL packet delay | Observed End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |
| >> Primary RAN Path Redundant Transmission Experience (UE to Primary NG-RAN) | Redundant Transmission Experience value of primary RAN path. |
| >> Secondary (Redundant) RAN Path Redundant Transmission Experience (UE to secondary NG-RAN) (NOTE 3) | Redundant Transmission Experience value of secondary RAN (redundant) path. |
| >> Primary N3 tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 4) | Redundant Transmission Experience value of primary N3 tunnel. |
| >> Secondary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 4) | Redundant Transmission Experience value of secondary N3 tunnel. |
| > Redundant Transmission Status | Redundant Transmission Status, i.e. redundant transmission was activated or not activated for the time slot entry. |
| > Ratio | Percentage on which UE, any UE, or UE group experience the packet drop rate and packet delay. |
| > List of primary NG-RAN nodes | List of Primary NG-RAN nodes identified based on current UE location |
| > List of secondary NG-RAN nodes (NOTE 5) | List of secondary NG-RAN nodes identified based on current UE location. |
| NOTE 1: The Observed Redundant Transmission Experience can be further derived by SMF from the observed UL/DL packet drop rate GTP-U and UL/DL packet delay GTP-U. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and only applicable when Target of Analytics Reporting is for a single UE. NOTE 3: in Case that the redundant transmission is established only on N3 or N3/N9, this value will be N/A. In case of end to end redundant UP paths, the UPF reports the packet delay of the two UP paths respectively to the NWDAF. NOTE 4: In case of I-UPF, this value includes both N3 and N9 experience. NOTE 5: In Case that the redundant transmission is established only on N3 or N3/N9, this value will be N/A. |  |

Table 6.13.3-2: Redundant Transmission Experience predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE or, any UE, a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the estimated Redundant Transmission Experience predictions applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Predicted Redundant Transmission Experience | Predicted Redundant Transmission Experience related information during the Analytics target period. |
| >> UL/DL packet drop rate GTP-U (NOTE 2) | Predicted UL/DL packet drop rate on GTP-U path on N3 (average, variance). |
| >> UL/DL packet delay GTP-U (NOTE 2) | Predicted UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |
| >> E2E UL/DL packet drop rate | Predicted End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |
| >> E2E UL/DL packet delay | Predicted End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |
| >> Primary RAN Path Redundant Transmission Experience (UE to Primary NG-RAN) | Predicted Redundant Transmission Experience value of primary RAN path during the Analytics target period. |
| >> Secondary (Redundant) RAN Path Redundant Transmission Experience (UE to secondary NG-RAN) | Predicted Redundant Transmission Experience value of secondary RAN (redundant) path during the Analytics target period. |
| >> Primary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3) | Predicted Redundant Transmission Experience value of primary N3 tunnel during the analytics target period. |
| >> Secondary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3) | Predicted Redundant Transmission Experience value of secondary N3 tunnel during the analytics period. |
| > Ratio | Percentage on which the UE, any UE, or UE group may efficiently use the PDU session with redundant transmission. |
| >List of primary NG-RAN nodes | List of Primary NG-RAN nodes identified based on predicted UE location during the analytics period. |
| >List of secondary NG-RAN nodes | List of Secondary NG-RAN nodes identified based on predicted UE location during the analytics period. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The Predicted Redundant Transmission Experience can be further derived by the SMF from the predicted UL/DL packet drop rate GTP-U and UL/DL packet delay GTP-U and based on which the SMF can decide to start redundant transmission or not. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and only applicable when Target of Analytics Reporting is for a single UE. NOTE 3: In case of I-UPF, this value includes both N3 and N9 experience. |  |

* * * * Next Change * * * *