SA WG2 Meeting #S2-154	S2-2211003

Toulouse, France, 14 - 18 November 2022	(revision of S2-2210241)

Source:	Ericsson, Huawei, HiSilicon, Intel

Title:	KI#4, Conclusion update

Document for:	Approval

Agenda Item:	9.9.1

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Remove the EN for KI#4 conclusion.

# 1. Discussion

For the conclusion of KI#4 we have the following ENs

Editor's note:	The interim conclusion is subject to feedback of RAN WGs and will be revisited as per the feedback.

Editor's note:	How MBSR is made available with information related to its roaming operation in a VPLMN is FFS.

RAN3 responses provides the following for KI#1 and KI#4

RAN3’s feedback on point #1: For the non-roaming case, RAN3 assumes that the OAM configures the mobile IAB-node in the same way as a Rel-16/17 IAB-node. The OAM-based parameter configuration is out-of-scope for RAN3. Some parameters may also be configured by the IAB-donor as specified in TS 38.473 and TS 38.331. RAN3 further achieved the following agreement:

RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed.

The roaming case is out-of-scope for Rel-18 mIAB. Therefore, OAM-configuration and OAM-connectivity for roaming mobile IAB-nodes have not been discussed.

RAN3’s feedback on point #4: IAB-node roaming was not discussed in Rel-16/17, and it is out-of-scope in Rel-18. RAN3 can therefore neither confirm nor deny whether the integration/inter-donor-migration procedures will work in a VPLMN.

For the 2nd EN, the responses in S2-2210197/R3-226048 are not conclusive at the moment in terms of configuration needed for mobile IAB-node. But it seems clear that RAN does not consider roaming is part of the work scope. Based on this and considering IAB-node is rather a network entity, our view is that SA2 can use the existing (pre-)configuration approach as base for non-roaming and roaming without specifying any extra method (as already specified in the 4th bullet). If RAN WGs provide further information on the configuration, SA2 can further synch with RAN WGs. Thus the 2nd EN can be removed with a NOTE to indicate that configuration method for KI#1 can be applied for KI#4.

For the 1st EN and first bullet in the current conclusions in clause 8.4, it’s also related to the mobility procedure (or “baseline procedure for IAB-DU migration” in RAN3 LS term) for KI#3. SA2 can make further adjustment based on response from RAN3 when such is received.

# 2. Proposal

This paper proposes to remove the 2nd EN.

* Start of change *

## 8.4	Conclusions for KI#4

For KI#4, the interim conclusions are as follows:

-	Depends on RAN WG confirmation, MBSR (IAB-DU) can use IAB-node integration procedure to integrate into VPLMN to provide service.

NOTE 1: SA2 assumes the IAB-node integration procedure has no further normative work in the RAN WGs.

-	It is assumed that some roaming agreement for MBSR operation in VPLMN is in place in HPLMN, and the 5GC can make use of it for authorization of MBSR in VPLMN based on subscription information. The MBSR subscription data store the mobile IAB-Operation allowed indication.

-	AMF provided MBSR authorized indication to NG-RAN during UE Context setup/modification procedure based on MBSR subscription data.

NOTE 2:	Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined in normative phase.

-	The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN.

NOTE 3: Configuration mechanism conclusions for KI#1 are applicable to KI#4.

-	If the MBSR (IAB-UE) performs initial registration with the PLMN and accepted by the network acting as MBSR based on subscription information, and the information on how to obtain configuration information is available at MBSR. If the MBSR has no information for configuration in the serving PLMN and is not allowed to operate in the serving PLMN, it records the serving PLMN as not supporting operation as MBSR.

-	The AMF of the MBSR can indicate to the MBSR that it is not allowed to act as an IAB node in the RA as part of registration procedure via registration procedure (either reject or accept). Even after the MBSR is authorized as an IAB node in the initial registration, the AMF of MBSR that supports location based restriction of MBSR operation shall de-reregister the MBSR if the MBSR is out of location allowed to operate. The MBSR should store the corresponding RA TAIs and avoid attempts to access these PLMNs/TAIs.

NOTE 4: The mechanism applies to both roaming and non-roaming MBSR operations.

* End of changes *