3GPP TSG-WG SA2 Meeting #154	S2-2211013

# Toulouse, France, November 14 – 18, 2022		(revision of S2-2210603)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0244 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRU assisted LCS architecture and procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For KI#7, the following conslusions have been achieved after 153-E meeting: To support PRU, the conclusions are as follows: - For PRU Registration, the conclusions include: - UE sends UL NAS Transport including PRU Registration Request to AMF. - The PRU Registration is an LCS supplementary service message and has no new AMF impact. - AMF sends the PRU Registration Request to LMF and may include a UE verification indication indicating whether this UE can serve as a PRU. - PRU Registration Request includes PRU information. - PRU information may include PRU’s positioning capability, location information (if known), type and state of the PRU (e.g. mobile/static type, on/off state) - The PRU may register with multiple LMFs. e.g., for the case a PRU is in multiple LMF overlapped serving areas. - LMF may indicate to NRF the PRU information or may store PRU information locally based on operator policy. - LMF stores PRU information and may indicate to NRF the PRU existence in its NF profile. - When the PRU information is provided to the NRF: - The serving LMF invokes the Nnrf_NFManagement_NFUpdate service operation to store the PRU information in the LMF profile already stored in the NRF. - When other LMF needs to obtain the PRU information for a certain area, the LMF just invokes the Nnrf_NFDiscovery including NF type which is set to LMF and indicates the area. NRF will return the profile including PRU information of the LMFs covering the area. - The target UE serving LMF may obtain PRU information locally, or may obtain PRU information and a PRU serving LMF from an NRF. - A mobile PRU may lead to the update of registration at serving LMF and/or registration at a new LMF - A serving LMF of PRU obtains PRU location measurements as described in clause 5.4.5 in TS 38.305 [6] by triggering the existing procedures defined in clause 6.11 in TS 23.273 [5]: - To improve target UE positioning, the serving LMF of the target UE may obtain PRU location measurements from a serving LMF of PRU. To support Reference UE, the interim conclusion is as follows: - In this study, the Reference UE is the PRU. NOTE: The above Reference UE conclusion applies only to FS_eLCS_Ph3. Other uses of Reference UE as defined in other R18 studies do not require Reference UE to be a PRU. To align with the conclusion, it is proposed to update the LCS architecture and procedure in TS 23.273 to support the PRU assisted positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.2, update the LCS architecture to support PRU. - In clause 4.3.5~4.3.11, add impacts on the related network function entities. - In clause 5.1, update LMF selection to cover PRU serving LMF selection case. - Add clause 5.x to desribe the PRU assisted LCS fearture and clarify PRU information as per the conclusion. - Add 6.X to define the PRU assisted location service procedure as per the conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRU assisted positioning is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.2,,4.2.1, 4.2.2, 4.3.5, 4.3.6, 4.3.7, 4.3.8, 4.3.9, 4.3.10, 4.3.11, 5.1, 5.x(new), 6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G enhanced positioning area: see TS 22.261 [3].

5G positioning service area: see TS 22.261 [3].

LCS Client: entity that interacts with GMLC for the purpose of obtaining location information for one or more UEs. The LCS Client may reside in the UE.

For the purposes of the present document, the following terms and definitions given in TS 23.271 [4] apply:

Call Related: see TS 23.271 [4].

Codeword: see TS 23.271 [4].

Current Location: see TS 23.271 [4].

Deferred location request: see TS 23.271 [4].

DL Positioning: positioning of a target UE in which the+ target UE obtains downlink measurements for a 3GPP RAT.

Immediate location request: see TS 23.271 [4].

Last Known Location: see TS 23.271 [4].

LCS (LoCation Services): see TS 23.271 [4].

Local Co-ordinates: see TS 23.032 [8].

Local Location: location determined by Local Co-ordinate(s).

Location Estimate: see TS 23.271 [4].

Positioning Reference Unit (PRU): see TS 38.305 [9].

Pseudonym: see TS 23.271 [4].

Pseudonym mediation device: Functionality that verifies pseudonyms to verinyms.

RAT Independent Positioning: positioning of a target UE in which the target UE obtains measurements not related to a 3GPP RAT.

Requestor: see TS 23.271 [4].

Requestor Identity: see TS 23.271 [4].

Response Method: for LCS Client using the OMA MLP protocol. Detail see TS 23.271 [4].

Scheduled Location Time: a future global time (e.g. UTC) at which a UE is to be located.

Service Type: see TS 23.271 [4].

Target UE: see TS 23.271 [4].

UL Positioning: positioning of a target UE in which NG-RAN obtains uplink measurements of the target UE for a 3GPP RAT.

UL+DL Positioning: positioning of a target UE using both DL Positioning and UL Positioning.

Velocity: see TS 23.271 [4].

Verinym: see TS 23.271 [4].

# * * * * Next change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

PRU	Positioning Reference Unit

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

# * * * * Next change * * * *

## 4.2	Architectural Reference Model

### 4.2.1	Non-roaming reference architecture

Figure 4.2.1-1 shows an architectural reference model for 5GS LCS for a non-roaming UE and PRU in reference point representation.

Figure 4.2.1-1: Non-roaming reference architecture for Location Services in reference point representation

NOTE 1:	(R)AN represents NG-RAN, trusted non-3GPP access or untrusted non-3GPP access.

NOTE 2:	Reference point interface related to charging functionality is not shown in this specification.

NOTE 3: PRU refers to a Positioning Reference Unit as defined in TS 38.305 [9], clause 5.4.5.

Figure 4.2.1-2 shows an architectural reference model for 5GS LCS for a non-roaming UE and PRU in SBI representation.

Figure 4.2.1-2: Non-roaming reference architecture for Location Services in SBI representation

### 4.2.2	Roaming reference architecture

Figure 4.2.2-1 shows an architectural reference model for 5GS LCS for a roaming UE and PRU in reference point representation.

Figure 4.2.2-1: Roaming reference architecture for Location Services in reference point representation

NOTE 1:	(R)AN represents NG-RAN, trusted non-3GPP access or untrusted non-3GPP access.

NOTE 2:	Reference point interface related to charging functionality is not shown in this specification.

NOTE 3: PRU refers to a Positioning Reference Unit as defined in TS 38.305 [9], clause 5.4.5.

Figure 4.2.2-2 shows an architectural reference model for 5GS LCS for a roaming UE and PRU in SBI representation.

Figure 4.2.2-2: Roaming reference architecture for Location Services in SBI representation

# * * * * Next change * * * *

### 4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].

A limited set of UE positioning capabilities can be transferred to the 5GCN during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF).

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

#### 4.3.X PRU

The PRU may support some or all functions of a UE. Compared with a UE, a PRU additionally supports the following functions:

NOTE:	A PRU may support a reduced set of UE functions.

-	Support sends UL NAS Transport including PRU Registration Request which includes PRU information to AMF.

-	The PRU support de-registration with a serving LMF.

The PRU information contains the following aspects:

-	PRU’s Positioning Capabilities.

-	Location information if known.

-	Type: whether a PRU is static or mobile.

-	State: ON/OFF.

Editor's note:	The PRU information may be further updated based on coordination with RAN1.

Editor's note:	The meaning of the on/off state needs to be determined

# * * * * Next change * * * *

### 4.3.6	UDM

The UDM contains LCS subscriber LCS privacy profile and routing information. The UDM is accessible from an AMF, GMLC or NEF via the Nudm interface.

The UDM contains an indication of whether a UE is authorized to serve as a PRU in the UE subscription data.

# * * * * Next change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF.

-	Receive UL NAS Transport including PRU Registration Request from a UE. PRU Registration Request includes PRU information.

-	AMF may verify whether a UE can serve as a PRU based on UE subscription data.

-    AMF sends the PRU Registration Request to LMF and may include a UE verification indication indicating whether this UE is authorized to serve as a PRU.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

# * * * * Next change * * * *

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-    The PRU may register with multiple LMFs. e.g. for the case a PRU is in multiple LMF overlapped serving areas.

-	LMF may indicate to NRF the PRU information or may store PRU information locally based on operator policy.

-	LMF stores PRU information and may indicate to NRF the PRU existence in its NF profile.

-	When the PRU information is provided to the NRF:

-	The serving LMF invokes the Nnrf_NFManagement_NFUpdate service operation to store the PRU information in the LMF profile already stored in the NRF.

-	When other LMF needs to obtain the PRU information for a certain area, the LMF just invokes the Nnrf_NFDiscovery including NF type which is set to LMF and indicates the area. NRF will return the profile including PRU information of the LMFs covering the area.

-	The target UE serving LMF may obtain PRU information locally, or may obtain PRU information and a PRU serving LMF from an NRF.

-	A mobile PRU may lead to the update of registration at serving LMF and/or registration at a new LMF.

-	LMF may support authentication of a PRU.

-	Support de-registration of a PRU.

-	A serving LMF of PRU obtains PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the existing procedures defined in clause 6.11 of TS 23.273 [5]:

-	To improve target UE positioning, the serving LMF of the target UE may obtain PRU location measurements from a serving LMF of PRU.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

# * * * * Next change * * * *

### 4.3.X	Network Repository Function, NRF

The NRF may be used to store PRU existence inidication or PRU information in PRU serving LMF profile. Following functions may be performed by an NRF based on operator’s policy:

-	Support LMF NF profile update once receives the PRU existence indication or PRU information from PRU serving LMF.

-	Support PRU serving LMF(s) discovery based on request from targer UE serving LMF.

# 5	High Level Features

# * * * * Next change * * * *

### 5.4.1	General

An LCS client or AF may or may not be authorised to retrieve the UE location, e.g. for commercial use. UE LCS privacy is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed access to UE location information. UE LCS privacy can be supported via subscription and via UE LCS privacy profile handling.

With subscription, privacy preferences for a UE are stored in a UE LCS privacy profile as part of UE subscription data in the UDM and queried from the UDM by another NF such as GMLC or NEF. The UDM may also store the UE privacy profile in the UDR. In this release of the specification, subscription of privacy preferences is restricted to the Call/Session unrelated Class as defined in clause 5.4.2.2.3 and the PLMN Operator Class as defined in clause 5.4.2.2.4.

With UE LCS privacy profile handling, the UE and/or AF can provide and update part of the UE privacy profile and provide it to the network as an update to the UDR. In this release of the specification, UE LCS privacy profile handling is restricted to the Location Privacy Indication as defined in clause 5.4.2.3.

The UE LCS privacy profile is used to indicate whether LCS requests from LCS clients and AFs are allowed or disallowed, together with the POI as defined in clause 5.4.4.

NOTE:	In clause 5.4, even if the UE LCS privacy detail is only described for LCS client, the same detail is also applicable for AF, if no exception statement.

### LCS privacy is not applicable to location of a PRU by the serving LMF for the PRU.

# * * * * Next change * * * *

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Location is disallowed - Location is allowed (default) Time period when the Location Privacy Indication is valid |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE |
| PRU Indication | O | Indication of one of the following mutually exclusive settings: - UE is allowed to function as a PRU - UE is not allowed to function as a PRU (default) |

The Mobile Originating data is defined in table 7.1-1 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location) - Autonomous Self Location (UE can receive location assistance data) - Transfer to Third Party |
| List of Assistance Data Types for MO-LR | O | A list of one or more types of location assistance data that may be provided to the UE in the MO-LR procedure. |

Table 7.1-3: LCS broadcasting data for a UE subscriber

| Broadcasting Data | Presence | Description |
| --- | --- | --- |
| List of Assistance Data Types | O | A list of one or more types of location assistance data for which ciphering keys should be provided to the UE if requested by the UE when the assistance data is broadcast using ciphering. |

# * * * * End of changes * * * *