SA WG2 Meeting #S2-154	S2-2211029

14 – 18 Nov. 2022, Toulouse	(revision of S2-2211099, S2-2210562)

Source:	MediaTek Inc., Qualcomm

Title:	KI#4: Resolve ENs and Update the conclusion

Document for:	Approval

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the conclusions on KI#4

# 1	Proposal

It is proposed to adopt the following proposal into TR 23.700-08.

# 1	Discussion

## 1.1	Discussion on Alt. 1 and Alt. 2 for Localized Service Information in clause 8.4.2

In the SA2#153E, it has interim conclusion on two alternatives for the content of the Localized Service Information and this will be further discussed in SA#154 meeting. It is related to the below EN:

In Alt. 1, some points need to be considered:

-	for the information (a) Identifier/name of the Localized service, it is not clear what the Identifier/name of the Localized services means to the UE when the UE performs the automatic Hosting network selection.

-	for the information (d) and (e), there is no prioritization for the UE when the UE performs the automatic Hosting network selection. Following the logic of automatic PLMN/SNPN network selection in the spec, the UE is provisioned with the prioritized list by UE Home network. Therefore, the UE can determine how to select a network based on the prioritized list. However, with the list (d) and (e) in Alt. 1, the UE cannot perform network selection due to no prioritization for the UE to do so.

In Alt. 2, some points need to be considered:

- for the information (a) and (b), the UE is provisioned with a list of prioritized Hosting network (i.e. SNPN ID or GIN) or the UE is provisioned with the allowed CAG list for PN-NPN as Hosting network. However, without the Validity information (i.e., time and/or location), the UE does not know when and/or where to perform automatic Hosting network selection.

Comparing the above points for Alt. 1 and Alt. 2, it proposes the following proposal for the content of the Localized service information:

-	the prioritized list of the Hosting network (i.e. SNPN ID or GIN) with the Validity information (i.e. time and/or location) in the case of SNPN as Hosting network

-	the allowed CAG ID list, in the case of PNI-NPN as Hosting network and the PNI-NPN is associated with CAG ID with the Validity information (i.e. time and/or location)

Proposal 1:

-	the prioritized list of the Hosting network (i.e. SNPN ID or GIN) with the Validity information (i.e. time and/or location) in the case of SNPN as Hosting network

-	the allowed CAG ID list, in the case of PNI-NPN as Hosting network and the PNI-NPN is associated with CAG ID with the Validity information (i.e. time and/or location)

## 1.2	ENs in the clause 8.4.2 for the content of the Localized Service Information

Based on the Proposal 1, the ENs related to Alt. 1 can be removed since the UE is provisioned with the prioritized list of the Hosting network with the Validity information. No additional information is needed for the UE to perform the automatic Hosting network selection.

For Alt. 2, the following ENs (EN 2-4 and EN 2-5) can be resolved based on the Proposal 1:

Editor's note 2-4:	It is FFS whether the UE receives list a/list b per localized service ID, or the lists are used for localized services in general.

Editor's note 2-5:	It is FFS whether necessary to associate validity conditions with the list in a and b above, and what are the conditions, e.g. time and/or location.

Proposal 2: The above two ENs can be removed directly based on the Proposal 1 since the list a/ list b are for Localized service in general and Proposal 1 includes the Validity information.

For EN 2-6 below:

Editor's note 2-6:	It is FFS what localized service information needs to be provided to UE in b above, in the case PNI-NPN as hosting network does not support CAG.

If a PNI-NPN does not support CAG, the UE can access to the PNI-NPN using network slicing mechanism and, e.g., based on different S-NSSAI/DNN for different services as specified in D.2 of TS 23.501. Furthermore, the Validity information should be provisioned along with the PNI-NPN ID to the UE to inform the UE when and where to perform PLMN selection. The details can be determined by CT WG1

Proposal 3: A NOTE is added:

-	NOTE X: 	If a PNI-NPN does not support CAG, the UE can access a PNI-NPN using network slicing mechanism as specified in D.2 of TS 23.501. In addition, the UE needs to perform PLMN selection for a PNI-NPN based on the Validity information. The details can be determined by CT WG1.

For EN 2-7 below:

Editor's note 2-7:	Alt. 1 provides UE with necessary information as a first step, and then the UE based on the received information derives how to perform other activities in later steps, e.g. network selection, credential determination and provisioning, etc. Alt.2 provides UE with information directly used for network selection, but requires that UE/network knows beforehand whether the desired localized service is provided by the hosting networks in the lists.

Proposal 4: It proposes to go with the Proposal 1.

## 1.3	ENs in clause 8.4.3

Proposal 5: Based on the changes made in the clause 8.4.2, it removes the ENs related to Alt. 1

For ENs for Alt. 2 below:

Editor's note:	It is FFS whether home network, may broadcast in a SIB an indication that localised service(s) are available in the geographic locality of the cell that is broadcasting.

Editor's note:	Whether SIB can include localized service information for hosting network selection is FFS and it shall be evaluated by SA3 on if it is feasible from security aspect.

Based on the Proposal 1, the UE is provisioned with the prioritized list of Hosting network with the Validity information by UE Home network, and the UE can know when and where a Hosting network can be selected for the Localized services. No need for UE Home network broadcasts an indication that the Localized service(s) are available in the UE current location.

Proposal 6: The above ENs can be removed directly.

Editor's note:	It is FFS whether, in the case of SNPN as hosting network, the hosting network may broadcast the supported localized service information (e.g. service identifiers and/or human readable service information) to help UEs discover the service and the hosting network. The localized service identifier may be pre-configured in the UE as per clause 8.4.2. The human readable service information is used for manual hosting network selection. UE may acquire this information from hosting network also via on-demand SIB while UE is RRC_IDLE or RRC_INACTIVE state in serving network.

For the above EN, it can follow the similar logic as specified in clause 5.30.2.4.3 of TS 23.501, the UE can provide to the user the list of SNPN hosting network the UE has respective credentials and related human-readable name for the manual network selection. The UE does not need to acquire such information via on-demand SIB.

Proposal 7: For the above EN, it can be removed by adding

-	Following same logic as in clause 5.30.2.4.3 of TS 23.501, the UE can provide to the user the list of SNPN hosting network the UE has respective credentials and related human-readable name for the manual network selection.

For below EN, based on the changes in clause 8.4.2, after the UE has registered to a Hosting network, the Hosting network can provide the information related to establish the PDU Session to access the Localized services. If the information is related to automatic Hosting network selection, the SoR procedure can be used as concluded in KI#5.

Editor's note: It is FFS whether the UE can query for further information on the localized service information hosted by the hosting network during the initial registration.

Proposal 8: The above EN can be removed directly, the UE does not need to do so.

## 1.4	ENs in clause 8.4.5

For EN below:

Editor's note 5-1:	It is FFS whether an extension of the Rel-17 onboarding mechanism is needed to support provisioning of credentials to use for localized services. Onboarding configuration information (which Includes PVS address(es) stored in UPF/AMF/PCF may need to be stored corresponding to different Localized services.

To address the above EN, it needs to consider Rel-17 Onboarding mechanism as follows:

-	if Onboarding network is a PLMN, the UE follows the procedures as specified in clauses 5.30.2.10.3 and clause 5.30.2.10.4.4 of TS 23.501.

-	if Onboarding network is SNPN, and the UE has the subscription of this SNPN, it should follow the similar procedures as Onboarding network is a PLMN, i.e. the UE performs the normal registration to the SNPN and User Plane Remote Provisioning are used.

-	if Onboarding network is SNPN, and the UE only has Default UE credentials,

-	if the Hosting network is SNPN, the UE could follow the clause 5.30.2.10.2 and clause 5.30.2.10.3 of TS 23.501 to obtain the subscription/credentials of SNPN Hosting network.

-	if the Hosting network is PLMN, the UE should have a USIM to access the network.

Based on the above, there is no need to enhance the Rel-17 onboarding mechanism. This EN can be removed without any change.

Proposal 9: based on the above, the EN can be directly removed.

* * * * Start of First Changes * * * *

### 8.4.2	Conclusion for the content of the localized service information

The UE is provided with:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services with, for each entry in the list, the Validity information (i.e. time and/or location), in the case of SNPN as hosting network and/or

b.	(existing) allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID with, for each entry in the list, the Validity information (i.e. time).

NOTE X: 	If a PNI-NPN does not support CAG, the UE can access a PNI-NPN using network slicing mechanism as specified in D.2 of TS 23.501. In addition, the UE needs to perform PLMN selection for a PNI-NPN based on the Validity information. The details can be determined by CT WG1.

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

* * * * Start of Second Changes * * * *

### 8.4.3	Conclusion for from where and how UE obtains the localized service information

The information for localized service and hosting network discovery, selection and access can also be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope.

5.	The information for localized service and hosting network discovery, selection and access can be preconfigured in the UE or dynamically provisioned by the hosting network or home network (via the VPLMN when roaming).

6.	In the case of SNPN as hosting network, the dynamic provisioning of prioritized list of hosting network information can be done via SoR.

NOTE:	How SOR-AF and/or UDM acquires hosting network information is outside the scope for 3GPP.

i.	The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply:

-	UE location as part of Registration procedure.

-	UE subscription data change, e.g. via external parameter provisioning.

NOTE X:	Following same logic as in clause 5.30.2.4.3 of TS 23.501, the UE can provide to the user the list of SNPN hosting network the UE has respective credentials and related human-readable name for the manual network selection.

7. When UE has accessed with the hosting network, the AMF can provide the latest localized service information to UE as part of Registration procedure (Mobility Update) or UE Configuration Update procedure.

8.	In the case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3].

* * * * Start of Third Changes * * * *

### 8.4.5	Conclusion for what credentials are used to access hosting network and how to obtain them

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of SNPN as hosting network:

1.	The UE checks whether it is possible to re-use home network credentials to access the hosting network:

a.	If the hosting network related information indicates support of CH credentials, the UE determines that home network credential can be used if the SNPN ID of the hosting network is included in the SNPN priority lists associated with home network subscription.

b.	If the UE obtains hosting network selection information from the home network, the UE uses the credentials provided by the home network to access the hosting network.

2.	If new credentials for accessing hosting network are needed, the UE may use the existing credential onboarding mechanism as per Rel-17.

NOTE:	The method of delivering credentials needs to be evaluated by SA WG3.

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of PNI-NPN as hosting network:

3.	Only UEs equipped with a USIM configured with PLMN credentials can access a hosting network which is a PNI-NPN. When the UE requests to access the hosting network, the home PLMN credential(s) are used during authentication procedure.

* * * * End of Changes * * * *