SA WG2 Meeting #S2-154	S2-2211098

Toulouse, France, Nov 14 - Nov 18, 2022

Source: 	Ericsson

Title: 	KI #4: Conclusion update merge Alt1Alt2 and resolve CAG

Document for: 	Agreement

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a conclusion update for Key Issue #4, to merge alt1 and alt2, and also resolve CAG related Editor's notes.

# Discussion

## 1.	Introduction

This paper has analysed and compared Alt 1 and Alt 2 for the provisioning UE information for localized services and proposed a way to combine Alt 1 and Alt 2.

This paper has also proposals to address the CAG related open issues in relation with the hosting network PNI-NPN.

## 2.	Alt 1 vs. Alt 2

As stated in Editor's note 2-7, the two alternatives have different focus. Alt 1 focuses on firstly providing information to UE, so that UE can understand which network(s) providing access to the localized service(s) and then relying on other existing procedures to determine the network selection. While Alt 2 focuses directly on the network selection related provisioning.

Editor's note 2-7:	Alt. 1 provides UE with necessary information as a first step, and then the UE based on the received information derives how to perform other activities in later steps, e.g. network selection, credential determination and provisioning, etc. Alt.2 provides UE with information directly used for network selection, but requires that UE/network knows beforehand whether the desired localized service is provided by the hosting networks in the lists.

Beside the network selection related provisioning, Alt 2 has also mentioned that a mapping between localized services and hosting network ID is needed in the following NOTE.

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

Such mapping in principle has the similar function as Alt 1, as it is to make the UE to understand which network(s) providing access to the localized service(s). According to the discussion in pCR S2-2209005/S2-2209006 from SA2#153E, this mapping information is known to hosting network/home network/UE via provisioning from the home network. Without this mapping information, the priority list in Alt 2 does not indicate which localized service can be accessed by the networks in the list. The UE will not know under what circumstances to use the priority list, and the network will not know why need to provision UE with such priority list. So, basically the NOTE above in Alt 2 is to accomplish the similar function as Alt 1, where the mapping is formulated as the localized service information (i.e. localized service ID and the associated hosting network IDs). The provisioning of the mapping information to UE and network is addressed in Alt 1 in clause 8.4.3. The network selection related provisioning is addressed in different clauses for Alt 1 and Alt 2 (see table below). While the network selection related provisioning in principle applies only in the business model where the relationship between hosting network and home network exists (see details in pCR S2-2210310). In other business models, it is not suitable to use SoR procedure for the mapping information provisioning, neither for network selection related provisioning since UE does not use home network subscription/credential to access hosting network.

Regarding the following EN in Alt 2, as the priority list received by SoR basically reflects home network operator policy on how UE select hosting networks and business relationship between hosting networks and home network, so it can be agnostic to localized services, considering business models that home network may or may not know localized service. But as long as business relationship exist between home network and hosting network, the home network can determine the network selection priority list based on the agreement between the two network operators.

Editor's note 2-4:	It is FFS whether the UE receives list a/list b per localized service ID, or the lists are used for localized services in general.

So, it is proposed to convert this EN to a NOTE below:

NOTE:	The hosting networks on the priority list received by SoR reflects business relationship between home network and hosting networks and can be used for localized service in general.

|  | Alt 1 | Alt 2 |
| --- | --- | --- |
| Mapping between localized services to hosting network IDs | Clause 8.4.2 Alt 1 The content of localized service information | Clause 8.4.2 NOTE |
| Provisioning of the mapping to UE/network | Clause 8.4.3 Alt 1, bullet 1/2/3 | Not described in the conclusion |
| Network selection related provisioning | Clause 8.4.4 bullet 1.b. Based on the mapping information, for the UE to be able to determine if home network credential can be reused to access hosting network(s), UE can initiate SoR procedure indicating the hosting network IDs to home network. Then home network based on the business relationship with hosting network, can determine which hosting network is allowed for UE to access, and what is the priority of the hosting network in the response to the UE initiated SoR procedure. | Clause 8.4.3 Alt.2 bullet 5/6/8 |

Proposals:

It is proposed to merge the two alternatives as below:

1.	Use Alt 1 in clause 8.4.2 as basis to conclude the content of the mapping information between localized service(s) and the hosting network(s).

2.	Use clause 8.4.3 Alt 1 bullet 1/2/3 as basis to conclude the provisioning of the mapping information.

3.	Use clause 8.4.3 Alt 2 bullet 5/6/8 as basis to conclude the network selection related provisioning i.e. for automatic network selection.

NOTE:	The change in TR related to merge Alt1 and Alt 2 is done with user name "Ericsson User C1".

## 3.	CAG

The localized service information provided to UE should at least make UE understand which network(s) can provide the access to the desired localized service.

In case of PNI-NPN as hosting network, the localized service information should at least have the mapping between the PLMN ID(s) of the PNI-NPN(s) and the localized service. Then, the UE takes this mapping information into consideration, together with the priority lists for PLMN selection received from SoR procedure to select hosting network PNI-NPN.

CAG IDs are not necessary to be included in the localized service information, due to:

-	PNI-NPN as hosting network may or may not use CAG for access control. There is no reason to mandate the use of CAG in PNI-NPN for localized service.

-	CAG is mainly used for access control purpose. It is under control of the hosting network and does not need to be bonded to localized services. Hosting network should have the possibility to decide whether to apply CAG for localized service, and to choose/update the CAG IDs that are used for localized services.

-	There is an existing mechanism to provision the UE with CAG information. If the PNI-NPN is deployed by a PLMN operator (VPLMN) other than the HPLMN, it is up to the business agreement between the VPLMN and HPLMN regarding the CAG usage at VPLMN, and the update towards the UE. If the PNI-NPN is deployed by HPLMN, it is HPLMN that takes full control of updating UE with the proper CAG information.

-	Localized service information provisioning and access control (CAG) provisioning should be separated:

-	Localized service information is intended to make UE to understand which network providing access to which localized service. While the actual setup between the UE and the network regarding whether/how the UE is allowed to access the PNI-NPN hosting network should reuse existing procedures, such as SoR procedure to determine the priority of the PNI-NPN, UCU or other NAS procedures to determine the Allowed CAG list, etc. This setup between the UE and the network is based on the business agreement between VPLMN and HPLMN, or within HPLMN (in case the PNI-NPN hosting network is deployed by HPLMN).

-	If access control provisioning for the specific PNI-NPN hosting network is done prior to hosting network selection, automatic network selection can be enabled. Since the Allowed CAG list only determines which CAG IDs are allowed for the UE to access without any implication on priorities, so  taking CAG as input for automatic network selection does not give priority of one CAG cell over non-CAG cell or another cell using different CAG ID that are using the same PLMN ID. Other mechanisms are required to steer the UE to use the correct CAG cell (e.g. an RFSP that is set to an RRM policy prioritizing frequencies used for the localized service, and/or when a dedicated S-NSSAI is used for the localized service then NG-RAN can also use the awareness of the S-NSSAI in Allowed NSSAI as an input to RRM decisions).

-	If access control provisioning for the specific PNI-NPN hosting network is not done prior to hosting network selection (i.e. the UE has no valid CAG information to access the hosting network), manual network selection can enable the UE to select the hosting network, and get CAG information updated later. Manual selection to a CAG cell when the CAD ID is not present in the Allowed CAG list on UE, requires the CAG cell to broadcast that PLMN allows user to manual select the CAG ID, per existing procedure.

| Open issues: | Proposals: |
| --- | --- |
| Editor's note 2-2: It is FFS whether it is beneficial to provide CAG information within the localized service information, given that there is existing procedure to update UE with CAG information, and whether CAG ID is used to identify hosting network. It is also FFS whether CAG IDs needs to be associated with localized service when sent to the UE. | As discussed above, localized service information provisioning should be separated from the access control provisioning, the CAG ID does not identify the hosting network, and Allowed CAG list can be kept on a per UE basis, i.e. it is proposed to remove this editor's note and remove clause 8.4.2 Alt 1, e, ii: |
| Editor's note 2-6: It is FFS what localized service information needs to be provided to UE in b above, in the case PNI-NPN as hosting network does not support CAG. | No matter if PNI-NPN supports CAG or not for localized service, PLMN ID of the PNI-NPN hosting network is the basic information for UE to understand which network provides access to the localized services. Whether UE is allowed to access the PNI-NPN, existing procedure can be used separately for access control provisioning towards UE. So, it is proposed to remove the Editor's note with the following changes for Alt 2 in clause 8.4.2: Alt.2. The UE is provided with: a. a list of prioritized hosting networks (i.e. PLMN ID in case of PNI-NPN hosting network, SNPN ID or GIN in case of SNPN as hosting network) for localized services. b. allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID, using existing mechanism defined in clause 5.30.3.3 of TS 23.501 [3]. NOTE: The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, PLMN ID). |
| Editor's note 3-2b: It is FFS how UE can associate the Allowed CAG IDs to the localized service if sent without any association to the localized service. | It is proposed to remove this EN without further action, as there is no need to bind CAG ID with localized service. |
| Editor's note 3-2c: The trigger for the home network to send the updated Allowed CAG ID list to the UE is FFS. | It is proposed to convert this EN to a NOTE. NOTE: Subscription data change can be the trigger for home network to update UE Allowed CAG list, but it is up to the operator policy and agreement between operators to determine when/how to trigger the change of the subscription data. |

NOTE:	The change in TR related to CAG is done with user name "Ericsson User AB".

# Proposal

Add the following conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

## 8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

### 8.4.1	General

The conclusion for KI #4 is made for each component that is evaluated in clause 7.4.

When UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If Home network is SNPN, the Hosting network can be only SNPN

If the UE accesses the Hosting network using the other credentials rather than the subscription/credentials from the UE Home network, the determination of the subscription used to access the Hosting network is by implementation specific prior to automatic network selection as described in NOTE 1 of clause 5.30.2.4.2 of TS 23.501 [3].

### 8.4.2	Conclusion for the content of the information for accessing localized services

The following interim conclusions are reached.  Two types of information are needed for localized service.

Type 1.	The localized service information is intended as additional information to aid UE’s manual selection of hosting networks, which can be provided by a serving network to the UE.  The localized service information can include the following elements:

a.	Identifier/name of the localized service.

b.	Validity conditions for the localized service, e.g. duration of remaining service operation ,time and location.

d.	Hosting network related information, per hosting network in the case of SNPN as hosting network:

i.	hosting network identifier, e.g. SNPN ID, GIN;

ii.	Optionally, other hosting network related information in free text format (information can be e.g. PVS address for UE to obtain credentials to access hosting network, portal address, balance/service rate, quality of service, etc).

e.	Hosting network related information, per hosting network in the case of PNI-NPN as hosting network:

i.	PLMN ID of the PNI-NPN.

ii.	Optionally, other hosting network related information in free text format (information can be e.g. PVS address for UE to obtain credential to access hosting network, portal address, balance/service rate, quality of service, etc).

Type 2.	To enable automatic selection of hosting network, the UE is provided with hosting network selection information from home network as shown below:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services with, for each entry in the list, the Validity information (i.e. time and/or location), in the case of SNPN as hosting network.

b.	PLMN ID and optionally allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID with, for each entry in the list, the Validity information (i.e. time and/or location).

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

### 8.4.3	Conclusion for from where and how UE obtains the information for accessing localized service

The information for localized service and hosting network discovery, selection and access can also be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope.

The following interim conclusions are reached.

The Type 1 localized service information  is provisioned as below:

1.	The localized service information is formulated as application data as described in clause 8.4.2 that can be stored in the home network, or a serving network and provided to the UE when the UE accesses the serving network.

2.	The external parameter provisioning procedure in clause 4.15.6 of TS 23.502 [4] is extended to support the provisioning of localized service information, or the OAM method is used to provision the data. If GPSI is provided the data is stored per UE, and otherwise the data is stored as non-UE specific data, but related to a Localized Service.

3.	The localized service information can be preconfigured in the UE or dynamically provisioned via signalling. For dynamic provisioning, the AMF provides the localized service information to the UE, based on the UE request of the desired application data (e.g. by specifying the identifier or name of the localized service). The AMF retrieves the localized service information from PCF and provides the localized service information to the UE via a new UE policy using UCU procedure. The PCF retrieves the data from UDR.

NOTE:	How UE obtains localized service identifier or name to make the request for dynamic provisioning is out of 3GPP scope.

The hosting network selection information is provisioned as below:

5.	The information for hosting network selection and access can be preconfigured in the UE or dynamically provisioned by the hosting network or home network (via the VPLMN when roaming).

6.	In the case of SNPN as hosting network, the dynamic provisioning of prioritized list of hosting network information can be done via SoR.

NOTE:	How SOR-AF and/or UDM acquires hosting network information is outside the scope for 3GPP.

i.	The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply:

-	UE location as part of Registration procedure.

-	UE subscription data change, e.g. via external parameter provisioning.

NOTE:	Given the potential large content of SIB information for manual selection, on demand SIB solution as already defined in TS 38.331 [14] can be used. Details of this option are to be determined by RAN WG2.

7.	In the case of PNI-NPN as hosting network, existing SoR procedure is reused to provision UE hosting network selection information (in case of the PNI-NPN is deployed by other PLMN operator than HPLMN), and the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3].

NOTE:	Subscription data change can be the trigger for home network to update UE Allowed CAG list, but it is up to the operator policy and agreement between operators to determine when/how to trigger the change of the subscription data.

8.	In the case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3].

### 8.4.4	Conclusion for how the localized service information is used by UE

The following principles based on the evaluation in clause 7.4.4 are recommended for the normative work:

1.	If UE uses home network credential to access a hosting network:

a.	When the end user intends to access localized service and the validity conditions of localized service are met, the UE initiates hosting network selection using the hosting network selection information.

i.	For SNPN as hosting network, the UE can switch between PLMN selection and hosting network selection following Rel-17 specification for SNPN selection. How the UE switches among the network selections is up to UE implementation.

ii.	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN.

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection, including if a new selection mode is required, is up to CT WG1 to decide.

b.	Hosting network selection needs to be authorized by the home network, via UE initiated SoR procedure with SoR information including certain authorized criteria e.g. time. After the home network authorization, the UE is allowed to initiate hosting network selection, applicable for both automatic and manual hosting network selection.

i.	For manual hosting network selection, the UE presents available localized service information it has received in clause 8.4.3 to the end user.

NOTE 3:	If SA1 requires that 	the home network authorization is needed before the UE performs manual selection of a hosting network, such authorization can be added during normative work.

c.	When authorized criteria of the hosting network selection are no longer met, the UE stops hosting network selection.

2.	If the UE needs to obtain a new set of credentials/subscription to access the hosting network:

a.	It is up to UE implementation to decide how to switch to the new subscription profile for accessing hosting network.

3.	The UE determines SNPN access mode is activated/de-activated using implementation specific means as specified in existing Release 17, or using received localized service/hosting network assistance information as input.

### 8.4.5	Conclusion for what credentials are used to access hosting network and how to obtain them

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of SNPN as hosting network:

1.	The UE checks whether it is possible to re-use home network credentials to access the hosting network:

a.	If the hosting network related information indicates support of CH credentials, the UE determines that home network credential can be used if the SNPN ID of the hosting network is included in the SNPN priority lists associated with home network subscription.

b.	If the UE obtains hosting network selection information from the home network, the UE uses the credentials provided by the home network to access the hosting network.

2.	If new credentials for accessing hosting network are needed, the UE may use the existing credential onboarding mechanism as per Rel-17. With enhancements enabling UE to request PVS for subsequent provisioning of credentials for accessing the current hosting network, i.e. the UE indicates in PCO during PDU session establishment that the request is for retrieving PVS address(es) that enables access to the curent hosting network. The onboarding network responds with the corresponding PVS(s) based on local configuration to the UE (i.e. DCS provided PVS address is not used). The ON-SNPN being a hosting network is then also acting a SO-SNPN.

NOTE:	The method of delivering credentials needs to be evaluated by SA WG3.

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of PNI-NPN as hosting network:

3.	Only UEs equipped with a USIM configured with PLMN credentials can access a hosting network which is a PNI-NPN. When the UE requests to access the hosting network, the home PLMN credential(s) are used during authentication procedure.

### 8.4.6	Conclusion for how localized service is accessed in hosting network per agreed conditions

The following principles based on the evaluation in clause 7.4.6 are recommended for the normative work:

1.	Validity conditions provided to the UE as part of the localized service information can be used to restrict the UE's access of the hosting network.

2.	Existing methods, such as network slicing, forbidden area restriction, service area restriction, CAG, LADN, URSP rules can also be utilized to restrict UE's access, i.e. no need for additional normative work for access control in hosting network.

3.	In order to restrict access to a hosting network to a specific area, a hosting network operator may deploy and broadcast multiple hosting network IDs, i.e. SNPN IDs for SNPN case and CAG IDs in the case of PNI-NPN, in different areas depending on localized service area validity. Each localized service is mapped to a specific hosting network ID. Multiple localized service areas can be mapped to the same hosting network ID if their allowed service areas are the same. Validity conditions are also used by hosting network to restrict access.

*** END CHANGES ***