3GPP TSG-SA WG2 Meeting #154	S2-2211138

# November 14 – 18, 2022, Toulouse, France							      (revision of S2-2210395)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0131 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE-to-UE Relay Discovery has been studied and concluded to be supported in Rel-18 per TR 23.700-33 clasue 8.1, so this feature needs to be specified in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions and procedures for UE-to-UE Relay Discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-to-UE Relay Discovery not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.X (new), 6.3.2.X (new), 6.3.2.X.1 (new), 6.3.2.X.2 (new), 6.3.2.X.3 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

5.2.X	5G ProSe UE-to-UE Relay Discovery

For 5G ProSe UE-to-UE Relay Discovery, both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

The procedures for 5G ProSe UE-to-UE Relay Discovery with Model A/Model B are defined in clause 6.3.2.X.

Discovery integrated into PC5 unicast link establishment procedure is also supported.

* * * * 2nd Change * * * *

6.3.2.X	5G ProSe UE-to-UE Relay Discovery

6.3.2.X.1	General

5G ProSe UE-to-UE Relay Discovery is applicable to both 5G ProSe Layer-3 and Layer-2 UE-to-UE Relay Discovery for public safety use and commercial services. To perform 5G ProSe UE-to-UE Relay Discovery, the 5G ProSe End UE and the 5G ProSe UE-to-UE Relay are pre-configured or provisioned with the related information as described in clause 5.1.

A Relay Service Code (RSC) is used in the 5G ProSe UE-to-UE Relay Discovery, to indicate the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs. The RSCs are pre-configured or provisioned on the 5G ProSe UE-to-UE Relay and the 5G ProSe End UE as defined in clause 5.1. The 5G ProSe UE-to-UE Relay and the 5G ProSe End UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service based the policy as specified in clause 5.1. A 5G ProSe UE-to-UE Relay supporting multiple RSCs advertises the RSCs using multiple discovery messages, with one RSC per discovery message.

6.3.2.X.2	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

Depicted in Figure 6.3.2.X.2-1 is the procedure for 5G ProSe UE-to-UE Discovery with Model A.

Figure 6.3.2.X.2-1: 5G ProSe UE-to-UE Relay Discovery with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered other UEs in proximity (e.g. via a previous direct discovery or direct communication procedures). The 5G ProSe UE-to-UE Relay obtains the User Info ID of other UEs in proximity.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, list of User Info ID of the 5G ProSe End UEs and RSCs, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.X.

A 5G ProSe End UE monitors announcement messages from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

Editor's note:	Whether Layer-2 IDs of 5G ProSe End UEs are included in UE-to-UE Relay Discovery Announcement message will be aligned with the RAN WGs.

6.3.2.X.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.X.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.X.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of itself, RSC, and User Info ID of the discoveree 5G ProSe End UE (UE-2), and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.X.

A 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

2.	A 5G ProSe UE-to-UE Relay that matches the RSC sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of the discoverer 5G ProSe End UE (UE-1), User Info ID of UE-to-UE Relay, RSC, and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.X.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	The discoveree 5G ProSe End UE (UE-2) that matches the value of RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1), and User Info ID of itself, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.X. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays, it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	The 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC, and User Info ID of the discoveree 5G ProSe End UE (UE-2), and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.X.

5.	The discoverer 5G ProSe End UE (UE-1) performs UE-to-UE Relay selection.

Editor's note:	Whether Layer-2 ID of 5G ProSe End UE is included in UE-to-UE Relay Discovery Response message will be aligned with the RAN WGs.

* * * * End of Changes * * * *