3GPP TSG-SA2 Meeting #154	S2-2211139

# Toulouse, France, Nov 14 - 18, 2022											 (revision of S2-2210897)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0155 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5.2.X 5G ProSe UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds 5G ProSe UE-to-UE Relay Discovery based KI#1 conclusion in TR 23.700-33. In order to resolve the NOTE 2, it proposes to add an UE-to-UE Relay operation supported indication of Source UE/Target UE/U2U Relay in the Model A/B Discovery message. NOTE 2: Whether and how a Source UE and a Target UE indicate support of UE-to-UE Relay operation will be determined in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe UE-to-UE Relay Discovery is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay Discovery is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

* * * Next Changes * * *

5.8.X	Identifiers for 5G ProSe UE-to-UE Relay Discovery

5.8.x.1 General

The 5G ProSe UE-to-UE Relay discovery message contains two sets of elements, a direct discovery set(s) and a UE-to-UE discovery set.

-	The direct discovery set of elements have the same content as the 5G ProSe Direct Discovery message as defined in clause 5.8.1 in TS 23.304.

-	The UE-to-UE discovery set contains information to support discovery via a 5G ProSe UE-to-UE Relay..

5G ProSe UE-to-UE Relay only modifies the UE-to-UE set of the elements, and forwards the end-to-end elements during the discovery procedures.

The contents of the Model A discovery messages are defined in clause 6.3.2.x.2 and the contents of the Model B discovery messages is defined in clause 6.3.2.x.3.

* * * End of Changes * * *