3GPP TSG-SA2 Meeting #154	S2-2211143

# Toulouse, France, Nov 14 - 18, 2022					 (revision of S2-2210881)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0150 | rev | 01 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-to-UE Relay with Integrated Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In study, it was concluded to include Discovery integrated into PC5 unicast link establishment procedure. In sol#1 in TR23.700-33, how to setup PC5 link is not clear. Furthermore, it is also not clear how to handle with the sharing of the PC5 link between the Source UE and U2U Relay when the Source UE communicates with multiple Target UEs, and with the sharing of the PC5 link between the Target UE and U2U Relay when the Target UE communicates with multiple Source UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces the solution for Integrated Discovery support with UE-to-UE Relay based on sol#1 with several E.N. on potential issue on PC5 link setup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Integrated Discovery will not be supported as concluded during study phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X or 6.X.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes – ALL TEXT IS NEW * * *

## 6.x	5G ProSe UE-to-UE Relay Communication

### 6.x.y 5G ProSe Discovery integrated into PC5 unicast link establishment procedure

### 6.x.y.1	General

5G ProSe Communication via 5G ProSe UE-to-UE Relay with Discovery integrated into PC5 unicast link establishment procedure is supported.

For Discovery integrated into PC5 link establishment, when a UE allows a UE-to-UE relay to be involved in the Direct Communication Request to the other UE, the UE indicates it by including a relay_indication in the broadcasted Direct Communication Request message.

When a UE-to-UE relay receives a Direct Communication Request including a relay_indication, it decides whether to forward the message according to e.g. Relay Service Code if there is any, Application ID, operator policy per Relay Service Code, signal strength, and local policy.

### 6.x.y.2	Procedure

Figure 6.x.y.2-1 5G ProSe Discovery integrated into PC5 unicast link establishment procedure

0.	UEs are authorized and provisioned with parameters to use the service provided by the UE-to-UE relays. UE-to-UE relays are authorized and provisioned with parameters to provide service of relaying traffic among UEs.

1.	UE-1 wants to establish a unicast communication with UE-2 and broadcasts a Direct Communication Request.

The Direct Communication Request includes a relay_indication, source UE user info, target UE user info, Application ID, as well as Relay Service Code if there is any.

The relay_indication indicates whether UE-to-UE relay can forward the DCR message or not. It is also used to limit the number of hops of UE-to-UE relay by removing relay_indication in the DCR message from the UE-to-UE Relay.

NOTE 1:	The data type of relay_indication can be determined in Stage 3.

2.	When receiving DCR with relay_indication from UE-1, Relay (here, relay-1 and relay-2) may decide to participate in the procedure and broadcast a Direct Communication Request message in its proximity without relay_indication.

The Direct Communication Request message includes the source UE user info, target UE user info and Relay UE info (e.g. Relay UE ID) in the message.

The DCR message uses relay’s L2 address as the source Layer-2 ID.

Editor’s Note: It is FFS which additional parameter may be included in DCR in step 1 and step 2.

3.	When UE-2 receives a Direct Communication Request from one or multiple UE-to-UE relays, UE-2 select a UE-to-UE Relay which UE-2 will respond. UE-2 may select UE-to-UE relay according to the signal strength, local policy, operator policy per Relay Service Code if any.

NOTE 2:	If UE-2 also receives a Direct Communication Request directly from UE-1, it may choose to setup a direct communication link with UE-1 by sending the Direct Communication Accept message directly to UE-1 according to clause 6.4.3.1. And step4-8 in this figure will not execute.

4.	security establishment and PC5 link setup or link modification may happen between UE-2 and the selected UE-to-UE Relay (here relay-1), if needed.

Editor’s Note: It is FFS how to perform PC5 link setup or PC5 link modification between UE-2 and relay-1.

5.	UE-2 replies Direct Communication Accept message to relay-1.

DCA message includes the source UE user info, target UE user info and Relay UE info of relay-1 in the message.

6.	security establishment and PC5 link setup or link modification may happen between UE-1 and relay-1, if needed.

Editor’s Note: It is FFS how to perform PC5 link setup or PC5 link modification between UE-1 and relay-1.

7.	Relay-1 responds with Direct Communication Accept to the UE-1. The DCA message includes source UE user info, target UE user info, and Relay UE info of relay-1.

Editor’s Note: It is FFS which additional parameter may be included in DCA in step 5 and step 7.

8.	For L3 UE-to-UE relay case, IP address allocation and retrieval procedure may happen.

Editor’s Note: It is FFS how to get IP address without step 8 and how to support non-IP case.

### 6.x.y.2.1	5G ProSe Discovery integrated into PC5 unicast link establishment procedure via Layer-2 UE-to-UE Relay

Editor’s Note: Any additional update of procedure via Layer-2 UE-to-UE Relay, e.g. according to RAN’s decision, will be included here.

* * * End of Changes * * *