3GPP TSG-SA WG2 Meeting #154	S2-2211146

# Toulouse, France, Nov 14 – 18, 2022,										 (revision of S2-2210485)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0135 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support path switching for U2N relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper tries to realize the conclusion of R18 ProSe ph2 KI#2 (Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration) in TR23.700-33 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Realize the conclusion of R18 ProSe ph2 KI#2 (Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration) in TR23.700-33 clause 8.2, incouding: - Add a clause of path switching support for U2N relay - Add the criteria for path switching about U2N relay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | path switching for U2N relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1. Remove detail description, which will be covered in procedure clause. 2. Remove text related to KI#4, only addresses to KI#2. 3. First NOTE: replaces “will be defined by RAN WGs” with “is defined in TS 38.331 [16]” |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change (all new text) * * * *

5.X	Path switching between two UE-to-Network Relays

The 5G ProSe remote UE performs the path switching between two UE-to-Network Relays based on the URSP rules of the Remote UE traffic handling described in clause 6.5.4 and 5G ProSe Policy in clause 5.1.4 when the NG-RAN configured measurement thresholds and the criteria are satisfied or based on re-selection rules from application layer if any with following considerations:

-	The 5G ProSe remote UE shall first select the target path which has same type with the original path (i.e., 5G ProSe Layer-2 UE-to-Network Relay, 5G ProSe Layer-3 UE-to-Network Relay with N3IWF and 5G ProSe Layer-3 UE-to-Network Relay without N3IWF).

-	If above process fails, then this 5G ProSe remote UE re-evaluates the URSP or keeps evaluating the next RSD of the selected URSP for the target path selection.

If there are the re-selection rules from the application layer (e.g. provided by an application server), the 5G ProSe Remote UE may use the re-selection rules to discover the target 5G ProSe UE-to-Network Relay UE. If there are multiple discovered 5G ProSe UE-to-Network Relay UEs that satisfy the re-selection rules, the 5G ProSe Remote UE selects the target 5G ProSe UE-to-Network Relay based on the priority in the re-selection rules from the application layer.

The principles from the 5G ProSe Policy in clause 5.1.4 and URSP rules of the Remote UE traffic handling described in clause 6.5.4 take the higher priority than the re-selection rules from the application layer.

NOTE:	The measurement trigger criteria for path switch between two indirect network communication paths is defined in TS 38.331 [16].

The path switching procedures between two indirect network communication paths are as described in clause 6.5.X.

* * * * End of Changes * * * *