SA WG2 Meeting #154	S2-2211147

Toulouse, France, Nov 14 - 18, 2022                   	(revision of S2-2210538)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0139 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Communication Path Switching between Two UE-to-Network Relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G_ProSe_Ph2 studies the key issue for support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration; and it concuded the path switching between two UE-to-Network Realys will be included in Release 18 of TS 23.304. This CR is submitted to introduce the procedures for communication path switching between two UE-to-Network Realys. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures for communication path switching between two UE-to-Network Realys. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for communication path switching between two UE-to-Network Realys are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE (All New Text) ******************

6.5.X	Procedures for Communication Path Switching between Two UE-to-Network Relays

6.5.X.1	General

The communication path switching between two UE-to-Network Relays includes the following scenarios:

-	Layer-3 UE-to-Network Relay with N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-2 UE-to-Network Relay.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay with N3IWF.

6.5.X.2	Procedures for Communication Path Switching between UE-to-Network Relays

This clause specifies the communication path switching procedures between two UE-to-Network Relays.

When performing the path switching between two 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-2 UE-to-Network Relay, the application layer procedures are used for service continuity support which is out of scope of this specification.

When performing the path switching between two 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the mechanism specified for Local mobility anchor within untrusted non-3GPP access networks using MOBIKE as specified in clause 6.2.9 of TS 23.501 [4] is reused.

When Layer-2 Remote UE in CM-CONNECTED state switches the communication path to Layer-3 UE-to-Network Relay with N3IWF, Layer 2 Remote UE's PDU session(s) can handover to the indirect path via Layer-3 UE-to-Network Relay with N3IWF by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

When Layer-3 Remote UE switches the communication path from Layer-3 UE-to-Network Relay with N3IWF to an indirect path via Layer-2 UE-to-Network Relay, Layer-3 Remote UE’s PDU session(s) can handover to the indirect path via Layer-2 UE-to-Network Relay by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

Editor's note:	The procedures for Layer-2 Remote UE in CM-CONNECTED state switching the communication path to another Layer-2 UE-to-Network Relay will be aligned with RAN WG specifications.

****************** END OF CHANGES ***************