3GPP TSG-WG SA2 Meeting #154	S2-2211150

# Toulouse, France, November 14 – 18, 2022		(revision of S2-22010695)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3803 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Monitoring for Dynamic Satellite Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Xiaomi, CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of Satellite Edge Computing via UPF deployed on satellite is concluded in clause 8.1 of TR 23.700-27. According to the conclusion, a new clause is proposed to contain all changes related to satellite backhaul and move existing satellite backhaul texts to the new clause since the enhancments are not only about UP management (which is the main purpose of clause 5.8 of TS 23.501) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe QoS monitoring for dynamic satellite backhaul. - Move the existing clause 5.8.2.15 to clause 5.x.x |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - QoS monitoring for dynamic satellite backhaul is not supported. - The structure of satellite backhaul is not easy to read. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.X(new), 5.X.Y(new), 5.8.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 5	High level features

## 5.X	Support for 5G satellite backhaul

### 5.X.X	Reporting of satellite backhaul to SMF

If the AMF is aware that a satellite backhaul is used towards 5G AN, the AMF may report this to SMF as part of the PDU Session establishment procedure as described in clause 4.3.2 of TS 23.502 [3]. If AMF is aware that satellite backhaul category changes (e.g. at handover), the AMF reports the current satellite backhaul category and indicates the satellite backhaul category change to SMF.

Satellite backhaul category refers to the type of the satellite (i.e. GEO, MEO, LEO or OTHERSAT, DYNAMIC_GEO, DYNAMIC _MEO, DYNAMIC _LEO, DYNAMIC _OTHERSAT) used in the backhaul. Only a single backhaul category can be indicated.

If dynamic satellite backhaul is used by the NG-RAN, i.e. capabilities (latency and/or bandwidth) of the satellite backhaul change over time due to e.g., use of varying inter-satellite links as part of backhaul, the AMF notifies the SMF of the corresponding dynamic satellite backhaul category to serve the PDU Session.

NOTE:	It is assumed that the AMF can determine the Satellite backhaul category for the notification to the SMF based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul.

### 5.X.Y	QoS monitoring when dynamic Satellite Backhaul is used

If dynamic satellite backhaul is used, QoS monitoring can be used to obtain packet delay as specified in clause 5.33.3 of TS 23.501 [4].

If AMF informs SMF that the dynamic satellite backhaul is used, the SMF informs this to PCF. The PCF may trigger QoS monitoring as specified in clause 5.33.3 of TS 23.501 [4] if dynamic satellite backhaul is used.

Editor’s Note: In this case, whether PCF can request QoS monitoring for the backhaul only, i.e. without receiving delay values including RAN part delay, is FFS.

# * * * * Second change * * * *

#### 5.8.2.15	Void

If the AMF is aware that a satellite backhaul with long delay is used towards 5G AN, the AMF may report this to SMF as part of the PDU Session establishment procedure as described in clause 4.3.2 of TS 23.502 [3]. If AMF is aware that satellite backhaul category changes (e.g. at handover), the AMF reports the current satellite backhaul category and indicates the satellite backhaul category change to SMF.

Satellite backhaul category refers to the type of the satellite (i.e. GEO, MEO, LEO or OTHERSAT) used in the backhaul. Only a single backhaul category can be indicated.

NOTE:	It is assumed that the AMF can determine the Satellite backhaul category for the notification to the SMF based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul.

# * * * * End of changes * * * *