3GPP TSG-WG SA2 Meeting #154	S2-2211153

# Toulouse, France, November 14 – 18, 2022		(revision of S2-2211133)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3794 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of local switch via UPF deployed on satellite for GEO backhaul case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Local switch for GEO satellite backhual is included in the conclusion in the last meeting, - For UPF deployed on satellite acting as PSA case: Based on pre-configuration, the SMF establishes the N19 tunnel between the two UPFs and corresponding UPF-level N4 rules to support local switch. The IP address(es) in UPF address pools are utilized in the UPF-level N4 session to detect UE-to-UE communication packets which are sent to pre-configured N19 for local switch. - For UPF deployed on satellite acting as ULCL and local PSA case: SMF determines to activate local switching for UEs based on: - 1) AF request including IPs/DNNs of UEs which require UE-to-UE communications, which will re-use an existing NEF API and no impacts are foreseen; or - 2) Target IP reported by on-ground PSA UPF as current reporting mechanism defined in TS 23.501 [1]. SMF configures UL-CL/BP to route data traffic toward the L-PSA. SMF configures Local PSA(s) based on the IP address(es) of target UEs, then UE-to-UE communication packets are detected and sent to the internal interface and optional N19 for local switch. The local data forwarding rules on L-PSA can be configured based on 5GVN user plane handing mechanism. It is proposed to capture above conclusion in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new sub-clauses under high level features clause to support local switch for GEO backhual cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local switch is not supported in the GEO satellite backhaul case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y.0 (new) 5.X.Y (new), 5.X.Y.1 (new), 5.X.Y.2 (new), 5.X.Y.3 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 5	High level features

### 5.X.0 General

Satellite backhaul can be deployed in various configurations where the UPF resides on the ground, on gNB side of the N3 or on board the satellite or on the satellite ground station side of the N3. This clause specifies the behavior for the deployment case where the UPF is on board the satellite.

### 5.X.Y  Local switch for UE-to-UE communicaions via UPF deployed on GEO satellite

#### 5.X.Y.1 	General

The UE to UE traffic may be locally routed by UPF(s) deployed on satellite (i.e., through local switch) to the target UE without traversing back to the satellite gateway on the ground.

Local switching via UPF(s) deployed on satellite in this clause only applies on GEO satellite backhaul case and considers only DNNs and slices for 5G VN.

N19 tunnel may be established between two UPFs deployed on different satellites to locally switch UE to UE traffic.

Only a single SMF is supported for local switching and N19 forwarding, i.e. both UEs are served by the same SMF.

NOTE X:	The latency optimisation that can be gained by inter-satellite N19 between UPFs on different GEO satellites depends on the distance between the satellites that can be substantial, depending on the number of deployed satellites.

Clause 5.X.Y.2 describes the case of PSA UPF deployed on satellite, the clause 5.X.Y.3 describes the case of ULCL and local PSA deployed on satellite (PSA UPF is on the ground). Selection of PSA UPF or UL CL/BP/local PSA on satellite is described in clause 6.3.3.

# * * * * Second change * * * *

##### 5.X.Y.2	 Local switch with PSA UPF deployed on satellite

A combination of DNN/S-NSSAI is assigned by the operator to the services deployed on GEO satellite, the URSP configuration is described in TS 23.503 [45].

If SMF selects the UPF deployed on satellite as PSA of UE’s PDU Session, the SMF configures the UE’s N4 session to forward/detect packet to/from the internal interface same as the configuration for the 5GVN group member's N4 Session in clasuse 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

Based on pre-configuration, the SMF establishes the N19 tunnel between PSA UPFs and set corresponding group-level N4 session rules utilizing IP address(s) in UPF address pool to forward/detect packet to/from the N19 tunnel interface.

Editor’s NOTE: The difference of the group-level N4 session rules utilizing IP address(s) in UPF address pool in this clause and the group-level N4 session rules ones in clasuse 5.8.2.13.1 is for further discussion.

For establishing N19 tunnel between the UPFs onboard the satellite, the UPFs are controlled by the same SMF.

- To process packets between UE and servers residing in DN, SMF configures rules to route traffic via N6 as described in clause 5.8.2.13.1

- .

The group-level N4 session is per DNN and slice. The SMF can create, update or delete the group-level N4 Session, i.e., add or delete N4 rules, allocate or release the N19 tunnel resources based on operator deployment, e.g., based on GEO satellite’s planed obsolescence or new GEO satellite setup.

# * * * * Third change * * * *

##### 5.X.Y.3	 Local switch with ULCL/BP and local PSA UPF deployed on satellite

For UPF deployed on satellite acting as ULCL/BP/local PSA case, the PSA UPF anchored by UE PDU sessions is on the ground, if the UEs are served by the same SMF, the SMF determines to activate local switching and N19 forwarding for the UEs using the GEO satellite backhaul, based on:

1) AF request including IPs of UEs which require UE-to-UE communications;

Editor’s Note: UE list or collection provision will reuse a NEF API defined by the Edge_Ph2 topic.

2) Target IPs reported by on-ground PSA UPF as current reporting mechanism in clasue 5.8.2.11.7. SMF configures the on-ground PAS UPF to detect UL packets with destination IP addresses which belong to the current UPF address pool.

After SMF receiving IP addresses from AF or on-ground PAS UPF, if SMF determines that UEs in communications are using the GEO satellites backhaul and UEs are allowed to access the DNAIs corresponding to the GEO satellites, for each UE communicating with target UE(s) in the communication group, the SMF selects the UPF deployed on GEO satellite according to the DNAI as UL-CL/BP and L-PSA, and configures UL-CL/BP with the following rule:

- Route the data traffic received from the UE and destined to IP address(es) of the target UE(s) to the L-PSA.

- Route other data traffic received from the UE to the PSA of the UE's PDU Session.

The SMF configures the Local PSA with local forwarding rules to forward the data traffic to the target UEs directly. If the selected L-PSAs are different for the UEs in the communication group, N19 tunnel is established between the L-PSAs. For establishing N19 tunnel between the UPFs onboard the satellite, the UPFs are controlled by the same SMF.

The SMF regards the UEs in the communication group as members of a 5G VN group, and configures the local data forwarding rules on L-PSA(s) based on 5GVN user plane handing mechanism in clasuse 5.8.2.13.1 (Support for unicast traffic forwarding of a 5G VN).

Editor’s Note: It is for further discussion whether or not the configurations are different and needs additional description, if the UL-CL/BP and L-PSA are combined.

NOTE Z:	The selected UPF deployed on satellite can be inserted as ULCL/BP/L-PSA reusing existing ULCL/BP insertion procedures defined in TS 23.502 [3] or TS 23.548 [6].

# * * * * End of changes * * * *