3GPP TSG-SA WG2#154	S2-2211174

Toulouse, France, 14-18 November 2022	(was S2-2208868)

Source:	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon

Title:	Conclusions for KI#5

Document for:	Discussion/Approval

Agenda Item:	9.18.1

Work Item / Release:	FS_5MBS_ph2/ Rel.18

Abstract of the contribution: Proposes conclusions for KI#5.

# 1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-47:

# >>>>BEGINNING OF CHANGES<<<<

## 8.5	Coexistence with existing power saving mechanisms for capability-limited devices

The following principles are applied for normative work to allow UEs to receive  multicast/broadcast MBS data when they are using power saving mechanisms (e.g. eDRX, MICO with active time etc):

- Solution #14 is used as the basis for normative work with the following further clarifications:

-	The UE is configured by the AF via the service announcement about a session start time and a possible sequence of scheduled activation times when the AF may activate the MBS session and transmit MBS data.

-	At the session start time and the possible scheduled activation times, the UEs apply the procedures of Solution #14 to receive MBS data. This means that for an MBS multicast session, if the UE has already joined the MBS session as defined in TS 23.247 clause 7.2.1.3, at the possible scheduled activation times, IDLE UEs need to listen for paging requests and if paged by the network with group paging follow the existing procedures in TS 23.247 clause 7.2.5.2. In this case, how long the UE need to listen to paging is left up to UE implementation.

-	If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure as currently defined in TS 23.247 clause 7.2.1.3. Whether the UE performs MBS join procedure in advance and stays „joined“ or every time at activation time is left up to UE implementation.

- How to support NR capability-limited (RedCap) UEs in MBS will be decided in normative phase considering possible related decisions of RAN WGs

# >>>>END OF CHANGES<<<<