3GPP TSG-SA2 Meeting #154	S2-2211180

# November 14 –18, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0575 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF updates to assist resource monitoring of AI/ML-based services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the conclusion of KI#1, it has been agreed that Observed Service Experience, User Data Congestion, DN performance Analytics, UE Communication Analytics can be used by the AF as assistance information for Application AI/ML operation. However, in the current TS 23.288, there is no explict statement captures that AF is a consumer of UE Communication Analytics. And general description to suppot NWDAF-based procedures for assisting AIML-based services is also needed for Observed Service Experience. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Document that the AF is a consumer of UE Communication Analytics. Document general description of supporting NWDAF-based procedures to assist AIML-based services for Observed Service Experience. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect documentation of UE Communication Analytics exposure to AF and missing description of Observed Service Experience assisting AIML services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.4, 6.7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[X]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

* * * Next Change * * *

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for NWDAF providing Service Experience for an Application

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE". If the Target for Analytics Reporting is either a SUPI or an Internal-Group-Id the procedure in clause 6.4.6 applies. At the same time, for an Application ID, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter Information that may include one or more of the following as defined in Table 6.4.1-1 (Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, RAT type(s), Frequency value(s)), Analytics Reporting Information=Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID, Event Filter information), Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of trusted AF, NWDAF provides the Area of Interest as a list of TAIs to AF. In the case of untrusted AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into geographic zone identifier(s) that act as event filter for AF.

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 3:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

If the consumer NF is a PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2 of TS 23.503 [4].

If the consumer NF is an AF (e.g. MEC or other Application Server), it may use the Observed Service Experience related network data analytics to determine whether the user experience can be satisfied. If not, the AF may determine to adjust service parameters, e.g. for a video service this may be bit rate, frame rate, codec format, compression parameter, screen size, etc. to better match the network conditions and achieve better user experience.

If the consumer NF is SMF, PCF or AF/Application Server, it may take into account the Observed Service Experience analytics per UP path (i.e. UPF and/or DNAI and/or AS instance address as defined in Table 6.4.3-1) to perform the following procedures:

-	The consumer SMF determines to (re)selects UP paths, including UPF and DNAI, as described in clause 4.3.5 of TS 23.502 [3]. In addition, the SMF may (re)configure traffic steering, updating the UPF regarding the target DNAI with new traffic steering rules.

-	The consumer AF/Application Server determines to adjust service parameters, e.g. service parameters of video for adjustment may be bit rate, frame rate, codec format, compression parameter, screen size, etc., or service parameters for the AI/ML operations described in clause 6.40 of TS 22.261 [X]. In addition, the AF/ Application Server may provide an updated list of DNAI(s) for SMF to perform relocation when appropriate.

-	The consumer PCF may provide an updated list of DNAI(s) for SMF to perform relocation upon AF request.

* * *Next Change * * *

#### 6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management, traffic routing handling, RFSP Index Management, QoS improvement or Inactivity Timer optimization, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC or an AF.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF, SMF and UPF that corresponds to this application ID. NWDAF may also collect data from AMF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Communication".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Application ID;

-	Area of Interest.

-	an optional list of analytics subsets that are requested (see clause 6.7.3.3);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics.

-	Optional Preferred level of accuracy per analytics subset (see clause 6.7.3.3);

-	Optional preferred order of results for the list of UE Communications:

-	ordering criterion: "start time" or "duration",

-	order: ascending or descending;

-	Optionally, maximum number of objects;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

* * *End of Changes * * *