3GPP TSG SA WG2 Meeting #154		S2-2211199

November 14-18 2022, Toulouse, France

Title:	LS Response on Latency impact for NTN verified UE location

Response to:	S2-2210193 / R2-2211044

Release:	Rel-18

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2

Cc:	SA1, RAN1, RAN3, RAN

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA2 thanks RAN2 for their LS on Latency impact for NTN verified UE location. SA2 has the following answers to the 2 questions from RAN2.

Q1	Is there any constraint on the latency (from trigger to result) of the verification procedure?

Answer:

In Release 17 and 18, location verification for regulatory services (e.g. Public Warning System, Charging and Billing, Emergency calls, Lawful Intercept, Data Retention Policy in cross-border scenarios and international regions, Network access) can occur when a UE performs some access to an AMF or MME at a NAS level, such as for initial PLMN Registration or Attach, Registration update or TAU, Service Request, PDU session or PDN connection establishment. The associated NAS procedure is first completed and then the serving AMF or MME can initiate location verification for the UE from an LMF or E-SMLC, respectively. Because the initial NAS procedure is first completed, there is no real time restriction on the latency of the location verification. Hence a latency of more than 10 seconds could be tolerated. However, a long period of location verification is not preferred because it could interfere with power saving for UEs which need to access a PLMN for only very short periods, and would allow a UE that was not at an allowed location to obtain service from the PLMN that might violate regulatory requirements. Hence, SA2 requests that location verification be capable of being completed within a period of approximately 1 minute maximum and 30 seconds preferably.

Q2	Can the verification procedure be run independently from the targeted services (e.g. in parallel to prevent any set-up delay)? If not, what is the estimate of set-up delay?

Answer:

As indicated above, location verification is started after an initiating NAS procedure has been completed and would then run in parallel with any other UE related activity. SA2 is not aware of any constraint at a 5GC level that might impede or delay the location verification once started.

# 2	Actions

To RAN2

ACTION: 	SA2 asks RAN2 to take the above answers into account when defining support for UE location verification in Release 18.

# 3	Dates of next TSG SA WG2 meetings

SA2 #154-AH-E			16-20 January 2023		E-meeting

SA2#155			20-24 February 2023		Athens