3GPP TSG SA WG2 Meeting #154		S2-2211202

November 14-18 2022, Toulouse, France

Title:	LS on LPP message and UE event reporting over a user plane connection

Response to:

Release:	Rel-18

Work Item:	5G_eLCS_Ph3

Source:	SA2

To:	SA3, RAN2, RAN3, CT1, CT4

Cc:

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: S2-2210999

# 1	Overall description

SA2 has agreed for Release 18 to support location event reporting over a user plane connection by a UE to an LCS Client or AF for a deferred periodic or triggered 5GC-MT-LR.

SA2 also discussed the user plane connection between UE and LMF, which can be used to transfer LPP message and supplementary event report between UE and LMF.

Please refer to TR 23.700-71v1.1.0 clause 8.1 for further details.

SA2 has the following questions for RAN2, SA3, CT1, CT4 on supporting the user plane connection.

- For the user plane connection between UE and AF/LCS Client:

To SA3

SA2 asks SA3 to comment on security information and security procedures and protocols that might be used to establish the user plane connection and subsequently report location events over it. More details in attached agreed CR in S2-2210999. SA3 is asked to comment on this.

To RAN2, RAN3, CT1 and CT4

SA2 asks CT1 and CT4 to comment on suitable types of user plane address for an LCS Client or AF (e.g. such as an IP address and/or FQDN) and whether the transport protocols and application level messages to transfer location event reports. More details in attached agreed CR in S2-2210999. SA2 asks CT1 and CT4 to comment on this.

- For the user plane connection between UE and LMF:

To SA3

SA2 asks SA3 to comment on security information and security procedures and protocols that might be used between UE and LMF.

To RAN2, RAN3, CT1 and CT4

SA2 asks CT1 and CT4 to comment on suitable types of user plane address for the LMF supporting user plane positioning (e.g. such as an IP address and/or FQDN) and whether, which or what the transport protocols and application level messages to transfer LPP messages could be identified or defined by RAN2, RAN3, CT1 or CT4.

# 2	Actions

To SA3, RAN2, RAN3, CT1, CT4

ACTION: 	SA2 asks SA3, RAN2, RAN3, CT1 and CT4 to respond to the questions above.

# 3	Dates of next TSG SA WG2 meetings

SA2 #154-AH-E			16-20 January 2023		E-meeting

SA2#155			20-24 February 2023		Athens