3GPP TSG-WG SA2 Meeting #154 e-meeting 	S2-2211208

Toulouse, November 14th -18th, 2022	(revision of S2-2210986 was S2-2210717)

Source:	Nokia, Nokia Shanghai Bell, NTT Docomo, AT&T

Title:	KI#3 Updated evaluations and conclusions

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3/ Rel-18

Abstract:  This paper provides update to KI#3  evaluations and conclusions

# 1. Introduction/Discussion

This paper covers only the aspect of graceful slice termination aspect.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-41

# * * * * First change * * * *

## 8.3	Conclusions for KI#3

For the graceful and gradual termination aspect:

-	When a slice is to be decommissioned, and become no longer available for UEs in a network slice by a known time the operator determines, the PDU Sessions of the slice should be gracefully (for supporting UEs of the timing information) and gradually released (for no supporting UEs of the timing information):

-	If the UE supports the handling of timing information indicating information on time of network slice availability, the network may provide the timing information to the UEs so the UE knows in advance when a network slice ceases to be supported. In this case, the UE can take the necessary actions to prepare for the slice not becoming available.

-	In addition, the AMF, for non-supporting UEs and for the case of UE not performing any actions despite of the timing information provided by the network, may be triggered by the OAM to start gradually terminating PDU Session(s) associated with S-NSSAI subject to be terminated by the expected time. The AMF releases PDU Session(s), associated with the S-NSSAI subject to be terminated, based on operator’s policy available at the AMF.

# * * * * End of changes * * * *