3GPP TSG-WG SA2 Meeting #154	S2-2211222

# Toulouse, France, November 14 – 18, 2022		(revision of S2-2211215)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0253 | rev | 3 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce new feature: support of LCS in PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-71 v1.1.0 has documented the conclusion of key issue#2 “enhanced positioning architecture for NPN deployment”. It was agreed the following principles: - The Local AMF/Any AMF in NPN deployment is used to transfer the Non-UE Associated Network Assistance Data between the RAN and LMF, as described in solution 4 and solution 5. - For UE Associated Network Positioning procedure, the LMF sends the UE Associated Network Positing message towards the RAN via the serving AMF. RAN responds UE Associated Network Positioning message to LMF via the serving AMF. - The LMF notifies the UE location to GMLC directly, i.e. without going through the serving AMF. It is necessary to specify the new feature in the TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the new feature” enhanced positioning architecture for NPN deployment” in the TS 23.273. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of R18 eLCS study for “enhanced positioning architecture for NPN deployment” is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), 6.11.3, Annex x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all texts new) * * * *

5.x	Support of location service in PNI-NPN with signalling optimisation

Support of location service in PNI-NPN is based on the PNI-NPN description defined in clause 5.30.3, TS 23.501[x]. PNI-NPN architecture described in Annex X are supported.

Under the PNI-NPN architecture described in Annex X, the relevant Network Functions are defined as follows:

- Local AMF, an AMF type function deployed in the local network, it supports:

  - Funtionaliteis related to AMF service operation “NonUeN2MessageTransfer” within Namf_Communication service, defined in TS 29.518[x]

  - Hanlding “UL and DL NON UE ASSOCIATED NRPPA TRANSPORT”, defined in TS 38.413[y]

NOTE 1: local AMF can support other functionalities defined in TS 23.501[x].

NOTE 2: it is assumed only one local AMF is deployed in the local network.

- LMF, deployed in the local network, it supports:

  - Pre-configured with local AMF FQDN or communication address.

  - For DL UE associated NRPPa signalling, sends the signaling message to the serving AMF.

  - For DL NON UE associated NRPPa signalling, sends the signaling message to the local AMF.

- NG-RAN, deployed in the local network, it supports:

  - Pre-configured with local AMF FQDN or communication address

  - Establishes TNL association with local AMF.

- GMLC, deployed in the local network

When UE accesss the NG-RAN in the local network, during the registration procedure, NG-RAN selects the AMF in the public network. With appropriate configuration, local AMF cannot be selected as the serving AMF for the UE.

During the positioning procedure, if LMF determines network-based positioning method, the positioning procedure defined in clause 6.11.2 is used.

For both immediate MT-LR, and deferred MT-LR, when LMF determines UE location, LMF provides the UE location to GMLC, as defined in clause 6.3.1.

# * * * * Second change* * * *

6.11.3	Obtaining Non-UE Associated Network Assistance Data

Figure 6.11.3-1 shows a procedure which may be used by an LMF to support network assisted and network based positioning. This procedure is not associated with a UE location session. It is used to obtain network assistance data from a NG-RAN node (e.g. gNB or ng-eNB). The procedure may be based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN.

Figure 6.11.3-1: Procedure for Obtaining Non-UE Associated Network Assistance Data

NOTE: to support location service in the PNI-NPN, local AMF is used instead of serving AMF, as defined in clause 5.x.

1.	The LMF invokes the Namf_Communication_NonUeN2MessageTransfer service operation towards the AMF to request the transfer of a Network Positioning message to a NG-RAN node (gNB or ng-eNB) in the NG-RAN. The service operation includes the Network Positioning message and the target NG-RAN node identity. The Network Positioning message may request position related information from the NG-RAN.

2.	The AMF forwards the Network Positioning message to the target NG-RAN node indicated in step 1 in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

3.	The target NG-RAN node obtains any position related information requested in step 2.

4.	The target NG-RAN node returns any position related information obtained in step 3 to the AMF in a Network Positioning message included in an N2 Transport message. The target NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 2.

5.	The AMF invokes the Namf_Communication_NonUeN2InfoNotify service operation towards the LMF indicated by the routing identifier received in step 4. The service operation includes the Network Positioning message received in step 4. Steps 1 to 5 may be repeated to request further position related information from the NG-RAN.

# * * * * Third change (all texts new)* * * *

Annex X (Informative):

X.1	PNI-NPN architecture to support location service with signalling optimisation

Figure X.1 shows the PNI-NPN architecture to support location services with optimisation of signalling latency and privacy, with the corresponding functional descriptions are defined in clasue 5.x.

When UE accesss the NG-RAN in the local network, during the registration procedure, NG-RAN selects the AMF in the public network. With appropriate configuration, local AMF cannot be selected as the serving AMF for the UE.

Assuming NG-RAN 1 is the serving RAN of UE, NG-RAN 2 and NG-RAN 3 illustrated in the figure below is for positioning signal measurement, and only supports NON UE NRPPA signalling transmission.

N1

NG-RAN 1

Serving AMF

N2

LMF

NLx

NL1

UE

Uu

local network

Public network

network

UDM

N8

GMLC

LCS client

Le

NL2

NL6

local AMF

N2

NG-RAN 2

NG-RAN 3

N2

N2

NL1

Figure X.1 PNI-NPN architecture to support location service with signalling optimisation

# * * * * End of changes * * * *