3GPP TSG-SA WG2 Meeting #154 	S2-2211264

# Toulouse, France, November 14 – 18, 2022	(revision of S2-2211166)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0582 | rev | 3 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Horizontal Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.8 of TR 23.700-81, the proposal specifies the procedure of Horizontal Federated Learning among Multiple NWDAFs in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the detailed procedure of Horizontal Federated Learning among Multiple NWDAFs. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for Horizontal Federated Learning among Multiple NWDAFs will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE(All new text)

6.2X	Federated Learning among Multiple NWDAFs

6.2X.1	Description

This clause specifies how NWDAF containing MTLF can leverage horizontal Federated Learning technique to train an ML model, in which there is no need for raw data transferring (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas i.e. sharing of ML model and of the learning results among multiple NWDAFs (MTLF).

6.2X.2	Procedures

6.2X.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2X.2.1-1: General procedure for Federated Learning among Multiple NWDAF

0. 	The consumer, e.g., NWDAF containing AnLF, sends a subscription request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID and ML model filter information as described in TS 23.288 [5], the NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a normal MTLF without FL server capability.

1.	Server NWDAF sends a request to the selected Client NWDAFs that participate in the Federated learning including some parameters (such as initial ML model information, data type list, maximum response time window, etc.) to perform the local model training for Federated Learning.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF, e.g. the gradient.

The ML model information are exchanged between the Client NWDAF(s) and the Server NWDAF during the FL training process.

Editor’ Note: The services in step 1, 3, 5a and 6 to enable FL based ML model training is for FFS, and the diagram in figure 6.2X.2.1-1 will be updated accordingly.

4.	The Server NWDAF aggregates all the local ML model information retrieved at step 3,  to update the global ML model.

5a.	Based on the consumer request, the Server NWDAF updates the training status (an accuracy level) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

Editor’ Note: Further details on providing the accuracy of FL model process is for FFS.

5c.	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

7.	Each Client NWDAF updates its own ML model information based on the aggregated ML model information distributed by the Server NWDAF at step 6.

NOTE:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is finished, the globally optimal ML model information could be sent to the consumer for the inference directly or via the normal MTLF without FL capability.

END of CHANGES