3GPP TSG-SA WG2 Meeting #154	S2-2211272

# November 14 – 18, 2022, Toulouse, France							      (revision of S2-2210951)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0130 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE-to-UE Relay has been studied and concluded to be specified in Rel-18 per TR 23.700-33 clasue 8.1, so the high level description of this feature needs to be documented in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add high level descriptions of UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of high level descriptions of UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4.1, 4.3.1, 4.3.3, 4.3.5, 4.3.8. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

1	Scope

The present document specifies the Stage 2 of the Proximity based Services (ProSe) features in 5GS. 5G ProSe features consist of: 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-UE Relay.

5G ProSe Direct Discovery identifies that 5G ProSe-enabled UEs are in proximity using NR.

5G ProSe Direct Communication enables establishment of communication paths between two or more 5G ProSe-enabled UEs that are in direct communication range using NR.

5G ProSe UE-to-Network Relay enables indirect communication between the 5G network and UEs (e.g. for UEs that are out of coverage of the network).

5G ProSe UE-to-UE Relay enables indirect communication between two 5G ProSe End UEs.

Security aspects of ProSe in 5GS are defined in TS 33.503 [29].

* * * * 2nd Change * * * *

4.1	General concept

Proximity based Services (ProSe) are services that can be provided by the 5GS based on UEs being in proximity to each other.

The 5GS enablers for ProSe include the following functions:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe UE-to-Network Relay;

-	5G ProSe UE-to-UE Relay.

* * * * 3rd Change * * * *

4.3.1	UE

Any 5G ProSe-enabled UE may support the following functions:

-	Exchange of information for 5G ProSe Direct Discovery between 5G ProSe-enabled UE and the 5G DDNMF over PC3a reference point.

-	Procedures for 5G ProSe Direct Discovery of other 5G ProSe-enabled UEs over PC5 reference point.

-	Procedures for 5G ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-2 End UE.

-	Procedures to act as a 5G ProSe Layer-3 End UE.

-	Indicating 5G ProSe Policy Provisioning Request in UE Policy Container for Registration Request or UE triggered 5G ProSe Policy provisioning, which may request one or multiple types of policies/parameters as listed below:

-	Policy/parameters for 5G ProSe Direct Discovery;

-	Policy/parameters for 5G ProSe Direct Communication;

-	Policy/parameters for 5G ProSe Layer-2 Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-2 End UE;

-	Policy/parameters for 5G ProSe Layer-3 End UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay.

-	Receiving the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay (e.g. including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1) and 5G ProSe UE-to-UE Relay. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Reporting the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability.

* * * * 4th Change * * * *

4.3.3	PCF

In addition to the functions defined in TS 23.501 [4] and TS 23.503 [9], the PCF includes functions to provision the UE with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [9] clause 4.2.2, based on the policies/parameters requested by the UE listed in clause 4.3.1 in the 5G ProSe Policy Provisioning Request:

-	PC5 usage reporting configuration.

-	Authorization policy and parameters for 5G ProSe Direct Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-3 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay).

-	PDUID allocation with its validity timer.

-	Authorization policy and parameters for 5G ProSe UE-to-UE Relay Discovery and Communication.

The PCF may update the 5G ProSe policy and parameters to the UE under certain conditions.

When receiving the 5G ProSe Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [2].

* * * * 5th Change * * * *

4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for 5G ProSe Direct Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-Network Relay Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-UE Relay Discovery and Communication.

* * * * 6th Change * * * *

4.3.8	ProSe Application Server

The ProSe Application Server supports the following functionalities.

For 5G ProSe Direct Discovery:

-	Maintains permission information for the restricted 5G ProSe Direct Discovery using RPAUIDs;

-	Storage of ProSe Discovery UE IDs and metadata;

-	Mapping of RPAUID and PDUID for restricted 5G ProSe Direct Discovery;

-	Provisioning parameters for Group Member Discovery to UE.

-	Interaction with 5G DDNMF for 5G ProSe Direct Discovery, including:

-	Allocation of the ProSe Restricted Code Suffix pool, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Restricted Code Suffix, if restricted Direct Discovery with application-controlled extension is used.

For 5G ProSe Direct Communication:

-	Provisioning a path preference for 5G ProSe Services to UDR;

-	Provisioning parameters for 5G ProSe Direct Communication to UE.

For 5G ProSe UE-to-Network Relay service:

-	Provisioning parameters for 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-Network Relay Communication to UDR.

For 5G ProSe UE-to-UE Relay service:

-	Provisioning parameters for 5G ProSe UE-to-UE Relay Discovery and 5G ProSe UE-to-UE Relay Communication to UDR.

* * * * End of Changes * * * *