3GPP TSG-SA2 Meeting #154	S2-2211295

# Toulouse, France, Nov 14 - 18, 2022                                           revision of S2-2210367

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0124 | rev | 4 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer-2 link management over PC5 reference point for U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusion of U2U relay has been agreed in the TR 23.700-33. Both the communication of peer End UEs after Model A/B discovery or without Model A/B discovery (i.e. Discovery integrated into PC5 unicast link establishment procedure) are supported. And both Layer-2 UE-to-UE Relay and Layer-3 UE-to-UE Relay are supported. This CR adds the contents of Layer-2 link establishment messages for 5G ProSe UE-to-UE Relay based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the contents of Layer-2 link establishment messages for 5G ProSe UE-to-UE Relay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose communication for U2U relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 6.X.1 is introduced in CR 0132. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st  Change * * * *

6.4.3.x	Layer-2 link management over PC5 reference point for 5G ProSe UE-to-UE Relay

6.4.3.x.1	Common part for Layer-2 link management over PC5 reference point for 5G ProSe UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.X:

-	In step 3, the Direct Communication Request message includes:

-	Source User Info: the identity of the source 5G ProSe End UE requesting relay operation (i.e. User Info ID).

-	UE-to-UE Relay User Info: the identity of the UE-to-UE Relay provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure (i.e. User Info ID) if the 5G ProSe UE-to-UE Relay Discovery procedure has been performed.

-	Target User Info: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure or from the ProSe application layer (i.e. User Info ID).

-	(optional) Target unicast destination Layer-2 ID: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

-	(optional) ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the first hop PC5 link establishment.

NOTE:	The Security Information is defined by SA WG3.

Editor’s Notes:	Clarification for other procedures, e.g. link modification, link identifier update etc. can be added later.

6.4.3.x.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

Editor’s Notes:	Clarification only applied for Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay can be added later.

6.4.3.x.3	Layer-2 link management over PC5 reference point for 5G ProSe Layer-3 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.X.1:

-	In step 5, the Direct Communication Request message includes User Info ID of source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC and Security Information to the Target 5G ProSe Layer-3 End UE.

-	In step 7, the Direct Communication Accept message includes User Info ID of source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC, QoS Info, IP Address Configuration.

-	In step 9, the Direct Communication Accept message includes User Info ID of source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC, QoS Info, IP Address Configuration and IP address of Target 5G ProSe Layer-3 End UE if received before step 9.

Editor’s Notes:	Clarification only applied for other Layer-3 link management procedure over PC5 reference point for 5G ProSe Layer-3 UE-to-UE Relay can be added later.

* * * * End of Changes * * * *