SA WG2 Meeting #S2-154E	S2-2211318

14 - 18 November, 2022, Toulouse, France                   rev of S2-2210893

Source:	Xiaomi

Title:	update conclusion 8.4.4 for key issue#4

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the conclusion 8.4.4 for Key Issue#4.

1 Discussion

Observation 1: as this paper is based on the assumption that Alt.1 and Alt.2 are combined in conclusion 8.4.2.

Proposal 1: to remove the alternatives descriptions.

Observation 2: there are two different selections, one is localized service selection and another is hosting network selection. Normally the localized service is selected or desired by user or validity conditions are met, then to perform the hosting network selection to find an appropriate hosting network that can provide such desired localized service.

Proposal 2: to differentiate the localized service selection from hosting network selection as following:

Localized service selection is prior to the hosting network selection. Hosting network selection is performed based on the selected localized service.

Localized service selection can be selected automatically, e.g., when the validity conditions are met. And also can be selected manually by user based on the localized services provided by the corresponding hosting networks that are available to UE. No normative work is needed.

Observation 3: for manual hosting network selection, there is no need for additional authentication from home network. If UE gets the localized service information from home network, that means kinds of authorization.  In this paper it proposes to two separated parts, hosting network selection and localized service selection. So hosting network selection will be based on the selected localized service.

Editor's Note 4-1:	It is FFS whether the home network authorization is needed before the UE performs manual selection of a hosting network.

Proposal 3, to remove the bullet b and bullet i and Editor’s Note 4-1. The bullet i is captured in the manual localized service selection and hosting network selection.

2 Proposal

It is proposed to agree the proposed text for inclusion in TR 23.700-08.

*** Start of changes ***

8.4.4	Conclusion for how the localized service information is used by UE

The following principles based on the evaluation in clause 7.4.4 are recommended for the normative work:

Localized service may be selected before hosting network selection or may be selected after hosting network selection. How to select the localized service is up to UE implementation.

Automatic hosting network selection and manual hosting network selection are both supported.

1.	If UE uses home network credential to access a hosting network:

a.	When the end user intends to access localized service and the validity conditions of localized service are met, the UE initiates hosting network selection using the hosting network related information received as part of the localized service information as in principle 1 in clause 8.4.2, or using the lists as in principle 2 in clause 8.4.2.

i.	For SNPN as hosting network, the UE can switch between PLMN selection and hosting network selection following Rel-17 specification for SNPN selection. How the UE switches among the network selections is up to UE implementation.

ii.	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN.

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection is to be determined by WG CT1.

NOTE 2:	Details regarding priority list for hosting network selection, including if a new selection mode is required, is up to WG CT1 to decide.

b.	Hosting network selection needs to be authorized by the home network, via UE initiated SoR procedure with SoR information including certain authorized criteria e.g. time. After the home network authorization, the UE is allowed to initiate hosting network selection, applicable for both automatic and manual hosting network selection.

i.	For manual hosting network selection, the UE presents available localized service information it has received in clause 8.4.3 to the end user.

Editor's Note 4-1:	It is FFS whether the home network authorization is needed before the UE performs manual selection of a hosting network.

c.	When authorized criteria of the hosting network selection are no longer met, the UE stops hosting network selection.

2.	If the UE needs to obtain a new set of credentials/subscription to access the hosting network:

a.	It is up to UE implementation to decide how to switch to the new subscription profile for accessing hosting network.

3.	The UE determines SNPN access mode is activated/de-activated using implementation specific means as specified in existing Release 17, or using received localized service/hosting network assistance information as input.

*** End of changes ***