3GPP TSG-WG SA2 Meeting #154	S2-2211321

# Toulouse, France,  November 14-18,  2022	(revision of S2-2211027)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2074 | rev | 2 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Non-3GPP access for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CaleLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following was concluded on KI#2 (Support of Non-3GPP access for SNPN) in TR 23.700-08: - Access to SNPN services via Untrusted non-3GPP access network includes support for Credential Holder, UE Onboarding and Emergency services. Specifically: - When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a of TS 23.501 [3]. - UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following: - If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection. - Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with UICC, as defined in clause 6.3.6.4.2 of TS 23.501 [3]. - When UE registering to SNPN over Untrusted N3GPP access for UE Onboarding, UE may select a N3IWF in the SNPN which supports UE Onboarding by using an SNPN ID in the pre-configured ON-SNPN selection information to construct N3IWF FQDN. NOTE 1: The format of FQDN will be specified by CT WG4. - If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without connected to a N3WIF) with a PVS to obtain the SNPN credentials. - Access to SNPN services via Trusted non-3GPP access network is to be specified according to the principles described in Solution #3, including support for credential Holder, Emergency services and UE onboarding. Specifically: - ANQP is to be extended to support advertising of: - SNPN IDs and GINs corresponding to SNPNs with which 5G connectivity is supported and related parameters as described in clause 6.3.3.1. - Support for Emergency services. - Onboarding enabled indication. NOTE 2: The work on additional parameters in ANQP is to be kept internal to 3GPP. - When accessing SNPN services via Trusted or Untrusted non-3GPP access the UE needs to be able to construct a prioritized list of WLAN access networks by using enhanced WLAN Selection Policy (WLANSP) rules from ANDSP (currently supported only for PLMN in TS 23.402 [9]) or based on local configuration. - N3IWF and TNGF needs to be able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE, which is up to RAN3 to define. - Access to SNPN services via wireline access network and use of Credential Holder is to be supported by defining a new GCI including a "NID" - The NSWO procedure is to be extended to support UE authentication using SNPN credentials (applies both to SIM-based and non-SIM based credentials). It is expected that the impact is limited to the use of a SUCI format whose "realm" part enables routing of SWa requests from the WLAN AN to the NSWO in the SNPN's 5GC, which is already supported. NOTE 3: Additional conclusions for wireline access related to UE behind 5G-RG depends on the progress on the FS_5WWC_Ph2 study. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding support of non-3GPP access for SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of non-3GPP access for SNPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.7.4 SUPI and SUCI for 5G-CRG and FN-CRG support

The SUPI for a FN-CRG subscription shall, based on operator configuration, contain either an IMSI, as described in clause 5.9.2 of TS 23.501 [2], or a GCI (Global Cable identifier defined in clause 4.7.9).

The SUPI for a 5G-CRG subscription shall, based on operator configuration, contain either an IMSI, as described in clause 5.9.2 of TS 23.501 [2], or a GCI (Global Cable identifier defined in clause 4.7.9).

Only 5G-CRG whose SUPI corresponds to an IMSI may use 3GPP access to connect to 5GC.

A SUPI containing a GCI takes the form of a NAI where the user part is the GCI and the realm part is an identifier of the operator managing the subscription.

NOTE:	The realm part used to identify the operator managing the subscription can differ depending on whether the wireline access network belongs to a PLMN or SNPN. The NAI format for SUPI containing GCI for PLMN and SNPN is defined in TS 23.003 [14].

The SUCI provided by the 5G-CRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

The SUCI provided to the network for FN-CRG support always corresponds to a SUPI containing a GCI. This SUCI acts as pseudonym of the SUPI and the UDM performs a mapping to the SUPI that, depending on operator configuration, contains either an IMSI or the same GCI than in the SUCI.

As described in TS 23.003 [14], for both cases where the SUCI contains an IMSI or contains a GCI, the SUCI contains an identifier of the Home network i.e. an identifier of the operator managing the subscription.

NOTE: If the SUCI contains an IMSI, the identifier of the operator managing the subscription is carried in the MCC/MNC part of the IMSI as defined in TS 23.003 [14].