SA WG2 Meeting #SA2-154 	S2-2211337

Nov 14 – 18, 2022,

Source:	China Mobile, China Telecom, Huawei, HiSilicon, Xiaomi, Ericsson?,Tencent?, Nokia, Nokia Shanghai Bell,Samsung?,Interdigital,vivo?

Title:	Conclusion update for KI#1

Document for:	Approval

Agenda Item: 	9.19.1

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes conclusion update for KI1

# Discussion

# Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

## 8.1	Conclusions for Key Issue#1

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case,

The following aspects are concluded as principles for the normative work:

-	Those data streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs. In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, the existing URSP Rule evaluation framework can be reused. A traffic descriptor (e.g. an FQDN) for the XRM session will be used during URSP rule.

-	The procedure for AF session setup with required QoS, is reused for XRM applications (untrusted AFs) interacting with NEF. However, current Nnef_AFsessionWithQoS service shall be extended to allow the AF to provide information for multiple medias.

-	Normative impact to AF and NEF/PCF: extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide, at the same time, service requirements, alternative service requirements for GBR traffic, a common ID and QoS monitoring for multiple IP data flows associated to a multi-modal (XRM) application.

-	AF may provide QoS monitoring requirement for a group of flows at the same time to the NEF/PCF. PCF should support to generate the QoS monitoring policy for a group of flows at the same time.

-	AF may provide delay difference threshold requirement for a couple of flows and subscribe the notification from PCF.

-	AF may provide joint admission control requirement for a couple of flows and indicate the critical flow to 5GS. Joint admission control means to admit the flows only if all of the flows can be admitted or admit the flows only if at least all the flows labelled as critical can be admitted.

-	AF may provide joint QoS fulfilment requirement. The QoS for specific flow is considered fulfilled only if the service requirements of all the flows can be fulfilled or if the service requirements of at least all the flows labelled as critical can be fulfilled.

-	PCF generates policies based on AF requirement to support the following:

-	PCF performs the flow authorization.

-	PCF provisions QoS information considering the requirements provided by the AF for all data flows associated to a multi-modal (XRM) application, which is associated with the common ID.

-	Optional,PCF generates policies to support the joint admission control.

-	Optional,PCF generates policies to support group QoS fulfilment.

-	Optional, PCF adjusts the PDBs for group flows based on Alternative QoS parameters to guarantee the delay difference.

-	These policies above are enforced only according to the AF provided explicit requirements.

# * * * *end of change * * * *