3GPP TSG-SA WG2 Meeting #154	S2-2211347

Toulouse, France, Nov 14 – 18, 2022

# Title:	LS on UPSI handling at the UE

# Response to:	-

# Release:	Rel-17/Rel-16/Rel-15

# Work Item:	TEI17

Source:	SA2

To:	CT1

Cc:	CT3

Contact Person:

Name:	Belen Pancorbo

Tel. Number:

E-mail Address:	belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

SA2 has discussed a LS from CT3 on UE Policy Control with PCF re-selection during AMF relocation, see C3-224697, that describes a scenario where the UE Policy Association Establishment includes no list of PSIs, or ANDSP support or OS ID to the PCF. SA2 discussed this topic, while one of the solutions discussed is that the PCF considers the list of PSIs stored in the UDR as the list of PSIs stored in the UE, even if the UE provided no list of PSI, no ANDSP support and no OS ID, then it may happen that the PCF may request the UE to remove a Policy Section that is not stored in the UE.

Question 1: How will the UE react if the PCF provides a Policy Section that contains only the PSI and there is no Policy Section stored in the UE with that PSI?

Question 2: Will the same UE behaviour be expected for a Rel-15, Rel-16 and Rel-17 UE?

2. Actions:

To CT1

ACTION 1: 	SA2 kindly asks CT1 to provide a response to question 1 and question 2.

ACTION 2: 	SA2 kindly asks CT1 to inform, when available, the outcome of the discussion, on whether the UE always provide UE Policy Container to the network.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting Ad-Hoc   Jan 16 – 20, 2023 	Electronic

TSG-SA2#155 Meeting        Feb 20 – 24, 2023 	Athens, Greece