3GPP TSG-WG SA2 Meeting #154 	S2-2211399

Toulouse, France, November 14 – 18, 2022	(revision of S2-2210670, 11317)

Source:	Lenovo, Ericsson

Title:	KI#4: Conclusions update related to Hosting network credentials

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: this paper proposes to clarify for KI#4 the use of credentials for primary authentication in the Hosting network.

# 1. Introduction/Discussion

## 1.1. Using the Rel-17 Onboarding procedure

In TR 23.700-08 clause 8.4.5 the following statements are documented:

2.	If new credentials for accessing hosting network are needed, the UE may use the existing credential onboarding mechanism as per Rel-17.

Editor's note 5-1:	It is FFS whether an extension of the Rel-17 onboarding mechanism is needed to support provisioning of credentials to use for localized services. Onboarding configuration information (which Includes PVS address(es) stored in UPF/AMF/PCF may need to be stored corresponding to different Localized services.

This paper proposes to clarify this case further.

In one aspect, the UE needs to be configured with default credentials for Onboarding in order to decide to initiate the Onboarding procedure with the selected Hosting network. In one example, the Localized Service Provider (LSP) can configure the UE with such default credentials and the LSP would take care to configure the Onboarding network with the correct PVS details to be used.

Therefore, we propose to resolve the EN by documenting that the LSP provides the Default credentials to the UE and the LSP takes care to provide the correct PVS details to the ON-SNPN.

Proposal 1: It is proposed that the Onboarding mechanism is used in case that the UE is configured with Default credentials. The ON-SNPN acting as hosting network becomes also a SO-SNPN.

# 2. Text Proposal

Based on the analysis above, it is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

## 8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

### 8.4.5	Conclusion for what credentials are used to access hosting network and how to obtain them

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of SNPN as hosting network:

1.	The UE checks whether it is possible to re-use the home network credentials to access the hosting network by performing the following:

a.	If the UE uses CH/User controlled priority lists part of the Home network subscription profile or if the UE uses hosting network related information from the Home network (part of the localized service information) which indicates support of CH credentials, the UE determines that home network credential can be used to register with the selected Hosting network.

2.	If the UE has Default credentials and the UE determines that new credentials for accessing hosting network are needed, the UE uses the Default credentials for the onboarding mechanism with the ON-SNPN acting as hosting network as per Rel-17 with the following enhancements:

[ OPTION 1 START

a.	The UE indicates in the PDU Session Establishment request for the restricted PDU Session that:

-	the request is for retrieving PVS address information that enables access to the current ON-SNPN network for localized services; and

-	If the UE is preconfigured with PVS address information for the hosting network, the UE also includes the PVS address information.

The SMF based on local configuration and policies determines which PVS address information to sends to the UE. The ON-SNPN acting as hosting network becomes also a SO-SNPN.

[ OPTION 1 END]

[ OPTION 2 START]

a.	In case that the UE is preconfigured with PVS address information for the hosting network and the UE receives PVS address information from the SMF during the PDU Session Establishment Accept message, the UE may determine based on local configuration whether to apply or ignore the PVS address information provided by the SMF.

[ OPTION 2 END]

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of PNI-NPN as hosting network:

3.	Only UEs equipped with a USIM configured with PLMN credentials can access a hosting network which is a PNI-NPN. When the UE requests to access the hosting network, the home PLMN credential(s) are used during authentication procedure.

# * * * * End of changes * * * *