SA WG2 Meeting #SA2-154 	S2-2211403

Nov 14 – 18, 2022,

Source:	China Mobile, China Telecom, Huawei, HiSilicon, Xiaomi, Ericsson,Tencent, Nokia, Nokia Shanghai Bell, Samsung, Interdigital,vivo

Title:	Conclusion update for KI#1

Document for:	Approval

Agenda Item: 	9.19.1

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes conclusion update for KI1

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

8.1	Conclusions for Key Issue#1

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case,

The following aspects are concluded as principles for the normative work:

-	Those data streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs. In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, the existing URSP Rule evaluation framework can be reused. A traffic descriptor (e.g. an FQDN) for the XRM session will be used during URSP rule.

-	The procedure for AF session setup with required QoS, is reused for XRM applications (untrusted AFs) interacting with NEF. However, current Nnef_AFsessionWithQoS service shall be extended to allow the AF to provide information for multiple medias.

-	Normative impact to AF and NEF/PCF: extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide, at the same time, service requirements, alternative service requirements for GBR traffic, a common ID and QoS monitoring for multiple IP data flows associated to a multi-modal (XRM) application.

-	AF may provide QoS monitoring requirement for a group of flows at the same time to the NEF/PCF. PCF should support to generate the QoS monitoring policy of flows at the same time.

-

-	PCF generates policies based on AF requirement to support the following:

-	PCF performs the flow authorization.

-	PCF provisions QoS information considering the requirements provided by the AF for all data flows associated to a multi-modal (XRM) application, which is associated with the common ID.

-	These policies above are enforced only according to the AF provided explicit requirements.

# * * * *end of change * * * *