3GPP TSG-SA WG2 Meeting #154	S2-2211409

# Toulouse, France, Nov 14 – 18, 2022,										 (revision of S2-2211292)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0135 | rev | 3 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support path switching for U2N relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper tries to realize the conclusion of R18 ProSe ph2 KI#2 (Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration) in TR23.700-33 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Realize the conclusion of R18 ProSe ph2 KI#2 (Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration) in TR23.700-33 clause 8.2, incouding: - Add a clause of path switching support for U2N relay - Add the criteria for path switching about U2N relay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | path switching for U2N relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: 1. Remove detail description, which will be covered in procedure clause. 2. Remove text related to KI#4, only addresses to KI#2. 3. First NOTE: replaces “will be defined by RAN WGs” with “is defined in TS 38.331 [16]” Rev 2: 1. Change the re-selection rules to information 2. Change the application information for target relay selection to NOTE Rev 3: Editorial modified the first paragraph |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.X	Path switching between two UE-to-Network Relays

If the NG-RAN configured criteria are met, the 5G ProSe Remote UE performs the path switching between two UE-to-Network Relays based on the URSP rules of the 5G ProSe Remote UE traffic handling described in clause 6.5.4, 5G ProSe Policy in clause 5.1.4, or information from application layer (if available) with following considerations:

-	The 5G ProSe Remote UE shall first select the target path which has same type with the original path (i.e., 5G ProSe Layer-2 UE-to-Network Relay, 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, and 5G ProSe Layer-3 UE-to-Network Relay without N3IWF).

-	If there is no same type target path, then this 5G ProSe Remote UE re-evaluates the URSP or keeps evaluating the next RSD of the selected URSP for the target path selection.

NOTE 1:	The NG-RAN configured criteria for path switch between two indirect network communication paths is defined in TS 38.331 [16].

NOTE 2:	If there is the information from the application layer (e.g. provided by an application server), the 5G ProSe Remote UE may use this information to discover the target 5G ProSe UE-to-Network Relay UE. If there are multiple discovered 5G ProSe UE-to-Network Relay UEs, the 5G ProSe Remote UE selects the target 5G ProSe UE-to-Network Relay based on the priority, if any, in the information from the application layer.

The principles from the 5G ProSe Policy in clause 5.1.4 and URSP rules of the 5G ProSe Remote UE traffic handling described in clause 6.5.4 take the higher priority than the information from the application layer.

The path switching procedures between two indirect network communication paths are as described in clause 6.5.X.

* * * * End of Changes * * * *