SA WG2 Meeting #S2-154	S2-2210173

14 - 18 November, 2022, Toulouse, France

SA WG2 Meeting ##153-e	S2-2208138

10-17 October 2022, Electronic Meeting

3GPP TSG-CT WG4 Meeting #111-e	C4-224636

# E-Meeting, 18th – 26th August 2022

# Title:	LS on Packet Delay Failure Threshold

# Release:	Rel-17

# Work Item:	TEI17, 5G_URLLC

Source:	CT4

To:	SA2

Cc:	CT3

Contact Person:

Name:	Yong Yang

Tel. Number:

E-mail Address:	frank.yong.yang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: C4-224543 (postponed)

1. Overall Description:

CT4 discussed a contribution (C4-224543) which proposes to add Packet Delay Failure Threshold parameter over N4 in PFCP protocol, so that the UPF can use it to determine if to send a QoS Monitoring Failure report. The contribution is based on the following requirement in 3GPP TS 23.503.

" 6.1.3.21   QoS Monitoring to assist URLLC Service

...

-             if the reporting frequency is periodic, the reporting period;

-             threshold for reporting packet delay measurement failure;

..."

However, during the discussion, it was pointed there is another contradictory requirement in 3GPP TS 23.503 as below:

" -   If the Reporting frequency indicates "event triggered", the Reporting threshold(s) and the minimum waiting time shall also be included in the PCC rule. The Reporting threshold(s) indicates the measurement threshold for each of the included QoS parameter(s) to be measured, i.e. the UL packet delay, DL packet delay or round trip packet delay. When Reporting threshold(s) is exceeded, the UPF shall report to the SMF and the SMF shall report to the PCF or to the AF. If more than one value is received at one given point of time for UL packet delay, DL packet delay or round trip packet delay respectively, the SMF reports the minimum and maximum packet delays to the PCF or the AF. The SMF sends the first report when the Reporting threshold is exceeded and the minimum waiting time is applied for the subsequent report (if the threshold is exceeded after the waiting time). The Reporting threshold(s) may also be used as the threshold for reporting packet delay measurement failure: if no measurement result is received for a delay exceeding this threshold, the UPF shall report to the SMF and the SMF shall report to the PCF or to the AF indicating a packet delay measurement failure.

"

CT4 assumes that the UPF shall use the threshold for reporting packet delay measurement failure to determine if to send a measurement failure report, since using reporting threshold would lead the AF/NEF receive only measurement failure reports.

CT4 also assumes that the threshold for reporting packet delay measurement failure is only corresponding to the round trip measurement.

2. Actions:

To SA2

ACTION:	CT4 kindly requests SA2 to confirm CT4's assumptions and update their specification if necessary.

3. Date of Next TSG CT WG4 Meetings:

3GPP TSG CT4#113			11/2022	Europe