3GPP TSG-WG SA2 Meeting #154	S2-2211234

# Toulouse, France, November 14 – 18, 2022		(was S2-2211014, S2-221086)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0245 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS Continuity During UE Mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The LCS continuity support during UE mobility feature in TS 23.700-71 has been concluded. This paper introduces the feature into the TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The basic framework for the different mobility scenarios is introduced. The paper focuses on 5GS and EPS handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LCS continuity during UE mobility feature in the study is not merged into the TS 23.273. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 4.3.3, 4.3.7, 5.X(new), 6.Y (new), 6.Y.1 (new), 6.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.3.3	Gateway Mobile Location Centre, GMLC

The Gateway Mobile Location Centre (GMLC) contains functionality required to support LCS. In one PLMN, there may be more than one GMLC.

A GMLC is the first node an external LCS client accesses in a PLMN (i.e. the Le reference point is supported by the GMLC). AFs and NFs may access GMLC directly or via NEF. The GMLC may request routing information and/or target UE privacy information from the UDM via the Nudm interface. After performing authorization of an external LCS Client or AF and verifying target UE privacy, a GMLC forwards a location request to either a serving AMF using Namf interface or to a GMLC in another PLMN using the Ngmlc interface in the case of a roaming UE.

The target UE's privacy profile settings shall always be checked in the UE's home PLMN prior to delivering a location estimate.

The "Visited GMLC" (VGMLC) is the GMLC, which is associated with the serving node of the target UE.

The "Home GMLC" (HGMLC) is the GMLC residing in the target UE's home PLMN, which is responsible for the control of privacy checking of the target UE.

Additional functions which may be performed by a GMLC to support location services include the following.

-	At an HGMLC, determine the serving AMF for a target UE when there is more than one serving AMF.

-	At an HGMLC, determine whether to attempt a second location request for a target UE from a different AMF when location information returned by a first AMF does not meet QoS requirements and there is more than one serving AMF.

-	At an HGMLC, support location requests from an external LCS client or NEF for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	At an HGMLC, forward location requests for a roaming UE to a VGMLC or serving AMF in the VPLMN based on deployment configurations.

-	At an HGMLC, receive event reports from a VGMLC or LMF for a deferred 5GC-MT-LR for periodic or triggered location and return to an external LCS Client or NEF.

-	At an HGMLC, support cancelation of a periodic or triggered location.

-	At an HGMLC, receive location information from an VGMLC for a 5GC-MO-LR and forwards to an LCS Client or an AF (via NEF) if requested by the UE.

-	At a VGMLC, receive location requests from an HGMLC for a roaming UE and forward to a serving AMF.

-	At a VGMLC, receive event reports from an LMF for a deferred 5GC-MT-LR for periodic or triggered location for a roaming UE and forward to an HGMLC.

-	At a VGMLC, receive location information from an AMF for a 5GC-MO-LR and forwards to an HGMLC.

-	At an HGMLC, reject the LCS request coming from a LCS client, e.g. when the number of Target UEs in the LCS request exceeds the Maximum Target UE Number of such client.

-	At an HGMLC, allocate the reference number for each location request from an external LCS client for LDR.

-	At an HGMLC, assign the pseudonym if pseudonym indicator is received in the service request and transfer it to external LCS client, e.g. when core network provides the UE's location information to LCS client. Resolve the verinym from the pseudonym, if it is received from the LCS client.

-	At an HGMLC, resolve group identifier to identifier of individual UEs and aggregate responses to LCS Client or NEF during bulk operation procedure.

-  During UE mobility between 5GS and EPS, the GMLC transfers the LCS request between 5GS and EPS, may map the LCS QoS and (re-)initiate the LCS sessions towards the 5GS to/from EPS.

* * * * Next Change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

- Provide the target MME ID to GMLC as part of Namf_Location_Response during 5GS to EPS handover to enable LCS session continuity

- * * * * Next change * * * *

### 5.X.1 LCS Continuity During UE Mobility

### LCS session continuity during UE mobility shall be supported. Continuity of LCS sessions using LCS procedures like shall be supported. Continuity of LCS sessions with the following attributes shall be supported

### - deferred triggered/periodic MT-LR, immediate MT-LR and MO-LR procedures

### - using uplink, downlink or uplink + downlink positioning methods

#### 5.X.1.1 Mobility Between 5GS and EPS

LCS continuity of UE in RRC-CONNECTED state shall be supported. There could be a need for mapping the LCS QoS from the source RAT to the target RAT.

The Lr’ interface between the EPC-GMLC and 5GC-GMLC is implementation specific – they may also be co-located.

# * * * * Next change * * * *

## 6.Y	Location Service Continuity between EPS and 5GS

Clause 6.Y.1 presents the location service continuity procedure from 5GS to EPS. Clause 6.Y.2 presents the location service continuity procedrue from EPS to 5GS. The GMLC in the Figure 6.Y-1 and Figure 6.Y-2 presents co-located EPC-GMLC and 5GC-GMLC or EPC-GMLC and 5GC-GMLC with the Lr' interface.

### 6.Y.1 Location Service Continuity from 5GS to EPS

Figure 6.Y-1: Location Service Continuity from 5GS to EPS

Step 1-4: MT-LR LCS session on 5GS as defined in step 1.a~step 12 of clause 6.1.2

Step 5-14: 5GS to EPS HO procedure defined in steps 1 ~ 12.b of clause 4.11.1.2.1 in TS 23.502 [19]

Step 15: AMF requests the LMF to cancel the LCS session on 5GS.

Step 16: LMF cancels the LCS session with the UE and NG-RAN

Step 17: LMF responds to the 5GC-GMLC with Namf_Location_ProvidePositioningInfo Response indicating failure of the LCS session with cause = HO to EPC while also including the target MME ID for the UE

NOTE 1: The LCS session cancellation with the UE and NG-RAN is to be completed before the UE is handed over to LTE. To enable this, the AMF could initiate steps 11, 15 and 17 in parallel.

Step 18: The 5GC-GMLC forwards the location request to the EPC-GMLC including all the parameters of the LCS Request and the target MME ID.

Step 19-24: The EPC-GMLC forwards the LCS Request to the Target MME received. The LCS session is started in EPS (same as steps 1-10 in Figure 9.18 in TS 23.271 [4]).

NOTE 2: EPC-GMLC could do a mapping of LCS QoS (from the 5GS defined values to EPS defined one). For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in EPS. For QoS attributes that are specific to 5GS, the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

Step 25-26:  The UE location estimate is returned by the EPC-GMLC to the 5GC-GMLC which then forwards the same to the LCS Client.

# * * * * Second change * * * *

### 6.Y.2 Location Service Continuity from EPS to 5GS

Figure 6.Y-2: Location Service Continuity from EPS to 5GS

Step 1~4: MT-LR LCS session on EPS defined in steps 1 to 3 of clause 6.1.2 in TS 23.271 [4].

Step 5~14: EPS to 5GS HO procedure defined in steps 1 to 16 in clause 4.11.1.2.2.2 and steps 1-4 in Clause 4.11.1.2.2.3 in TS 23.502 [19].

Step 15: The MME initiates the cancellation of the LCS session over EPS

Step 16: e-SMLC cancels the LCS session with the UE and E-UTRAN

Step 17: The MME indicates failure to the EPC-GMLC with reason "HO to 5GC" and including the target AMF ID.

NOTE 1:  The MME could initiate steps 15 and 17 in parallel to step 11 to ensure that the LCS session cancellation happens before the handover completes while the UE is in EPS.

Step 18:  The EPC-GMLC forwards the location request to the 5GC-GMLC including all the parameters of the LCS Request and the target AMF ID including the QoS.

Step 19-24: The LCS Session starts as defined in in steps 1-23 clause 6.1.2.

NOTE 2:  There could be a mapping of LCS QoS (from the values defined for EPS to values defined in 5GS - this could be done by the GMLC.  For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in 5GS.  For QoS attributes that are specific to EPS, the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

Step 25-26: The UE location estimate is returned by the 5GC-GMLC to the EPC-GMLC which then forwards the result to the LCS Client.

# * * * * End of changes * * * *