SA WG2 Meeting #155	S2-2302336

# Athens, Greece, 2023-02-20 – 2023-02-24 												was S2-2301417

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0092 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNAI mapping based on conclusions in TR 23.700-48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], [Huawei], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR contains the description and procedure of DNAI mapping |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed description and procedure of mapping table between EAS IP address and DNAI Rev-2: The Key for storage and Key for retrieving (searching) should be the same, which means it is hard to have a range as a Key. DNAI mapping from only geographic area should be supported |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Absence of DNAI mapping procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR does not strictly follow the CR drafting rules, this to highlight the changes on the rev-1 version of the CR. In an approved version this will be corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE (all is new)

## 6.x	Support of mapping table between EAS IP address and DNAI

### 6.x.1	General

. . In order to make sure AF obtain target DNAI, the 5GS may help determine proper DNAI(s) and notify the information to AF based on AF request providing EAS IP address/EAS IP range or FQDN.

NEF/UDR is configured by OAM with the mapping information between EAS IP/IP range and DNAI. AF may subscribe the mapping information modification to NEF. AF may request immediate reporting. When the configuration information (relationship between DNAI and EAS IP address/IP address range) is changed, the UDR can notify the new mapping information to NEF. If mapping information is stored in UDR, NEF may subscribe to mapping information in UDR, where NEF may request immediate reporting. If subscribed to  information change, the UDR notifies the NEF (if the information is stored in the UDR), and the NEF notifies the AF.

The EAS address and DNAI Mapping Information record in UDR is shown Table 6.x.1-1.

Table 6.x.1-1 Description of EAS address and DNAI Mapping Information

| Parameters | Description |
| --- | --- |
| EAS address(es) | IP address(es) of the EASs or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es)) and/or FQDN(s) where the EAS is deployed for each DNAI. [Conditional] (NOTE 1) |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [Mandatory] |
| DNN | DNN for the EAS Deployment Information. [Mandatory] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [Optional] |
| Geographical Area | Geographical areas for EAS Deployment information. e.g. list(s) of TAIs and/or range of TAs per DNAI [Conditional] (NOTE 1) |
| NOTE 1: At least one of EAS address or Geographical Area shall be present in the record. |  |

Editor’s Note: how the above parameters can be used as key/sub Key is FFS.

### 6.x.2	AF request for DNAI Procedures

Figure 6.x.2-1: AF request for DNAI based on AF request

1.	AF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range or FQDN and/or geographical area and optionally: DNN, S-NSSAI, and AF Identifier.

2.	If mapping information is stored in UDR, the NEF determines the DNN and, if needed, the S-NSSAI related to the AF identifier. If geographical area is received, NEF translates this to a TA and  invokes the Nudr_DM_Subscribe service to subscribe to  DNAI(s) mapping information. If mapping information is stored in NEF, skip step 2-3.

3.	UDR notifies with the DNAI mapping information.

4.	NEF determines the suitable DNAI(s) based on the EAS address(es) and/or geographical area.

5.	NEF notifies the DNAI(s) or the updated DNAI information to AF. If DNAI information is stored in the UDR and subscription to DNAI information change , steps 3 to 5 take place.

## END of CHANGES