3GPP TSG-WG SA2 Meeting #155 	S2-2302665

Athens, Greece, February 20 – 24, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Functionality and Procedure of Ranging SL Positioning Control

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the description on functionality and procedure of Ranging SL Positioning Control based on the KI#4 conclusion in TR 23.700-86.

1. Introduction/Discussion

This paper proposes to add the description on functionality and procedure of Ranging SL Positioning Control based on the KI#4 conclusion in TR 23.700-86.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (all new)* * * *

## 5.X	Ranging/SL Positioning Control

### 5.X.1	General

A Ranging/SL Positioning layer is introduced on the UEs (i.e., Target UE, Reference UE, SL Positioning Server UE) under application layer and above AS layer.

Functionalities supported by the Ranging/SL Positioning layer include

- to handle service request received from application layer and control the Sidelink Positioning and Ranging operation;

- discovery of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions;

- control signalling between UEs or among a group of UEs or between UE and LMF to manage and coordinate the Sidelink Positioning and Ranging operations.

- receive Application Requirements (e.g., one time or periodic ranging, ranging for distance or direction measurement or both) from application layer, and to provide these requirements to Access Stratum layer for AS layer operation.

The group management can be performed at application layer, and the application layer may provide group identifier information to the Ranging/SL Positioning layer.

Ranging/Sidelink Positioning Protocol (RSPP) is introduced for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE). Ranging/SL Positioning signalling related to RSPP is carried over PC5-U. V2X Communication procedures as defined in TS 23.287 [3] and 5G ProSe Direct Communication procedures as defined in TS 23.304 [4] are used for the RSPP communication between UEs over PC5. If PC5-U is used, Ranging/SL Positioning signaling is carried over V2X/ProSe Layer as payload. The handling of the Layer-2 ID and Application Layer ID information can reuse the V2X/ProSe transport mechanism defined in TS 23.287 and TS 23.304.

Editor’s note: Whether RSPP is over PC5-U or PDCP is used will be aligned with RAN WG decision.

A LMF may be involved when at least one of the Target UE and the Reference UE are in the network coverage. The LMF needs to be selected by the AMF based on the LMF Ranging/SL Positioning capabilities. When LMF is involved for coordination of SL Positioning, SL Reference UE in the same serving PLMN of Target UE is discovered and selected.

Editor’s note: The protocol used between UE and LMF will be decided by RAN, and the corresponding procedures will be aligned.

A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution and SL reference UE selection in case of out-of-coverage or for UE-only Operation if no Ranging/SL Positioning capable LMF is available. If LMF is capable for Ranging/SL Positioning and is reachable by Target UE and/or Reference UE, the LMF can still decide that SL Positioning Server UE executes the result calculation.

Multiple UEs may be involved in a single Ranging/Sidelink Positioning session.

Whether Ranging/Sidelink Positioning results are exchanged over the SR5 depends on the negotiation during the control signalling.

# * * * * Second change* * * *

## 6.X	Ranging/SL Positioning Procedure

Figure 6.X.1-1: Procedure for Ranging/SL Positioning

1.	UE1 and UE2 may get the ranging authorization policy and parameters from PCF during the registration procedure. The ranging authorization policy and parameters may include whether the UE is authorized as Reference UE or Target UE.

2.	UE1 gets the ranging request from the application layer, and the ranging request includes Application Requirements (e.g., one time or periodic ranging, ranging for distance or direction measurement or both).

3.	UE1 performs Ranging/Sidelink Positioning device discovery, as described in clause 6.Y.

4.	UE1 and UE2 perform the RSPP related procedures to exchange the coordination information, e.g., Ranging/Sidelink Positioning capability, Ranging/Sidelink Positioning assistant data.

Editor's note:	It is FFS whether additional procedure between step 3 and step 4 to initiate the ranging session is needed.

5.	The Ranging/SL Positioning layer of each UE provides the ranging configuration to the AS layer. The ranging configuration includes the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both.

6.	The ranging operation is performed in the AS layer of each UE.

7.	UE1 or UE2 performs the ranging result calculation, and the ranging result calculation may be shared between the UEs.

# * * * * End of changes * * * *