3GPP TSG-WG SA2 Meeting #155 	S2-2302930

Athens, Greece, 20-24 February 2023

# Title:	[Draft] Reply LS on Security architecture for 5G multicast/broadcast services

# Response to:	Reply LS on Security architecture for 5G multicast/broadcast services (S3-223919/S2-2302189)

# Release:	Release 17

# Work Item:	5MBS

Source:	SA2

To:	SA3

Cc:	CT3, CT4, SA4

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 0147 to TS 23.247

1. Overall Description:

SA2 thanks SA3 for their LS, where SA3 informs that SA2 about an agreed CR introducing a new Multicast/Broadcast Service Security Function (MBSSF). According to the CR, "the MBSSF is a logical function that includes the security functionalities defined in TS 33.246. MBSSF can be collocated with either MBSF or MBSTF and the according interfaces are up to the implementation of the deployment options."

SA2 would like to comment that the MBSSF, is now reflected in the stage 2 architecture in TS 23.247 based on the attached CR. SA2 expects that the related signalling procedures will be documented in SA3 specifications. The MBSSF needs to be inserted into the user plane path for MBS data distribution but SA2 expects that these procedures will not be standardised in rel.17.

As the result SA2 recommends that SA3 clarifies that the MBSSF needs to be (not only "can be") collocated with MBSF or MBSTF.

SA2 also understands that according to the agreed SA3 procedures a UE needs to discover key management servers  at the MBSSF. In TS 33.246 the key management servers are discovered from the service announcement (ref. TS 33.246, clause 6.3.2.1A). SA2 expects that SA3 will work with SA4 to document the related information in specifications under SA4 control. SA2 will not work on related discovery procedures in Rel-17 but leave them to SA3 and SA4 to document.

2. Actions:

To SA3:

ACTION: 	SA2 kindly ask to take the above information into account and provide further clarification.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #156	17-21 April 2023		e-meeting

TSG-SA2 Meeting #157	22-26 May 2023		EU