3GPP TSG-WG SA2 Meeting #155	S2-2302967

Athens, Greece, February 20 – 24, 2023

Source:	Interdigital

Title:	Proposed texts for located UE discovery and selection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL/Rel-18

Abstract of the contribution: This contribution proposes texts for high level description on located UE discovery and selection of TS 23.586v0.1.0.

# 1.	Discussion

Based on TR conclusion, there shall be a description on located UE discovery and selection.

# 2.	Text proposal

It is proposed to agree the following changes to TS 23.586:

# * * * * First change * * * *

## 5.X	UE Discovery & Selection

## 5.X.X	Located UE Discovery & Selection

Located UE discovery and selection functionality is supported by UE.

For network assisted ranging and SL positioning service, target UE discovers located UEs which are in the same serving PLMN of target UE.

For Mobile Terminated Location Request, LMF may ask target UE to discover located UEs. To assist the Target UE the LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE.

NOTE: The provisioning of candidate list of Located UE assumes that the capability and location information of the Located UE in the list are maintained by 5GC.

For Mobile Originated Location Request, target UE may trigger discovery procedure of located UE without request from LMF.

LMF may be involved in Located UE selection per RAN’s decision.

# * * * * End of change * * * *