3GPP TSG-SA WG2 Meeting #155	S2-2303548

Athens, Greece, Feb 20 – 24, 2023

Title:	LS on UE specific data and analytics exchange between HPLMN and VPLMN

Response to:	-

Release:	Rel-18

Work Item:	eNA_Ph3

Source:	SA2

To:	GSMA NRG, DESS, WAS

Cc:	SA3

Contact Person:

Name:	Xiaoyan Duan

E-mail Address:	duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

3GPP SA2 sent a LS (S2-2207142) to GSMA asking for possible feedback on data and analytics exchange in roaming case between HPLMN and VPLMN, but has not received any response yet.

SA2 has reached the following conclusions regarding possibility of data and/or analytics exchange between HPLMN and VPLMN (see 3GPP TR 23.700-81 clause 8.3):

-	VPLMN may consume analytics generated by HPLMN or request input data and generate analytics on its own:

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for Network Slice selection and SMF selection for PDU Session management.

-	UE-related analytics provided by the HPLMN (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

-	HPLMN may consume analytics generated by the VPLMN or request input data and generate analytics on its own.

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

-	An NWDAF is used as entry point to exchange analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent and it filters the information exposed in reply to the request.

However SA2 understands that neither user consent nor operator policies on roaming agreements are within the scope of SA2 to decide on: Work regarding user consent is under the SA3 remit and ongoing. It is SA2’s understanding that operator policies regarding roaming agreements fall under the GSMA expertise.

Among these analytics identified for exchange in roaming case so far:

- The Slice load level analytics is not UE-specific, and does not use UE-specific data as input for analytics, see clause 6.3.2A of TS 23.288.

- The NF load analytics is not UE-specific, and may or may not use UE-specific data (i.e. UE speed, UE Orientation, Per UE attribute (Destination, Route, Average Speed, Time of arrival) ) as input for analytics, see clause 6.5.2 of TS 23.288.

- The Service experience analytics can be non UE-specific (i.e. Service Experience for a Network Slice or an application) or UE-specific (i.e. Service Experience for a UE), and may or may not use UE-specific data (i.e. QoE metrics, UE location, Performance Data) as input for analytics, see clause 6.4.2 of TS 23.288.

Based on the reply SA2 received from SA3 (S3-223020) and the above analysis, SA2 would like to seek the views of GSMA on the following questions regarding UE specific data and/or analytics exchange between HPLMN and VPLMN:

- Does GSMA have any concerns or requirements (e.g. about being subject to local regulations, operator policies, user consent) on

  - UE specific data and/or analytics exchange between HPLMN and VPLMN, and

  - possible storage (and its duration) of such user data in the VPLMN?

2. Actions:

To GSMA NRG, DESS and WAS:

ACTION: 3GPP SA2 kindly asks GSMA to provide feedback on UE specific data and/or analytics exchange between HPLMN and VPLMN. Considering that 3GPP SA2 Rel-18 will be frozen in June 2023, it would be highly appreciated that GSMA provides their feedback before May 2023.

3. Date of Next SA2 Meetings:

SA2#156e	17th - 21st April 2023 		E-Meeting

SA2#157	22nd – 26th May 2023 		Berlin, DE